ML Inference for ARM64 Devices

(MiniOS ARM64 Unikernel)

Test Case Documentation

| Document Title | ML Inference for ARM64 Devices — Test Case Documentation |
| --- | --- |
| Document ID | MiniOS-TST-001 |
| SRS Reference | SRS v3.0 — March 18, 2026 |
| Version | 1.0 |
| Total Test Cases | 246 |
| Test Period | 10 March 2026 – 18 March 2026 |
| Target Platform | QEMU Virt – ARM Cortex-A53 (AArch64) |
| Submitted To | Dr. Romi Banerjee |
| Repository | https://github.com/Piyush2005-code/MiniOS.git |
| Status | 291 PASS / 10 FAIL (301 cases) |

Test Team

| Student ID | Team Member | Role (SRS §5.1) | Test Domain | Cases | Period |
| --- | --- | --- | --- | --- | --- |
| B24CS1020 | Darpan Baviskar | Integration Lead | HAL: UART · Timer · MMU | 55 | Mar 10–12, 2026 |
| B24CS1031 | Harshit | ML Runtime Developer | Types · Memory · Lib String · Cmd Framework · ULFS · Networking · ONNX Parser · ONNX Runtime | 124 | Mar 10–14, 2026 |
| B24CS1101 | Piyush Singh Bhati | Technical Lead | Status Codes · Scheduler | 74 | Mar 12–18, 2026 |
| B24CS1002 | Aashma | System Components Developer | Context · Exceptions · KAPI · System | 48 | Mar 13–18, 2026 |

Confidential — Internal Use Only

1. Document Control

1.1 Revision History

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| 1.0 | 18-Mar-2026 | Initial release — full test suite (246 test cases) aligned with SRS v3.0. Phase 2 additions: 55 new test cases (Lib String, Command Framework, ULFS, Networking, ONNX) bringing total to 301. | Harshit / Darpan Baviskar / Piyush Singh Bhati / Aashma |

1.2 Approvals

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Test Lead | Harshit |  | 18-Mar-2026 |
| Reviewer | Darpan Baviskar |  | 18-Mar-2026 |
| QA Owner | Piyush Singh Bhati |  | 18-Mar-2026 |

2. Introduction

2.1 Purpose

This document provides the complete test case specification and execution record for the ML Inference for ARM64 Devices project (codename: MiniOS). It serves as the authoritative reference for all functional verification activities conducted during the testing phase, covering all subsystems defined in SRS v3.0 — from HAL drivers through kernel memory management, cooperative scheduling, and system-level integration. The tests verify conformance with the 30 functional requirements (FR-001–FR-030) and supporting non-functional requirements defined in the SRS.

2.2 Scope

The test suite encompasses 301 test cases organised across 22 components, targeting a bare-metal AArch64 unikernel running on QEMU virt (Cortex-A53). The system is a specialised unikernel OS optimised for ML inference execution on ARM64 processors, with well-defined boundaries per SRS §1.3. Tests are categorised into three execution environments:

- QEMU Target Tests — executed directly on the emulated ARM64 hardware (UART, Timer, MMU, Context Switch, Exceptions, System tests)

- Host Unit Tests — compiled and executed natively on x86_64 using the Unity 2.6.2 test framework with hardware stubs (Types, Memory, Scheduler, Status Codes, KAPI)

Excluded from test scope (per SRS §1.3.2): POSIX compatibility, dynamic program loading, general-purpose filesystems, ML training, dynamic ONNX shapes, and multi-process environments.

2.3 Test Environment

| Parameter | Details |
| --- | --- |
| Emulator | QEMU 5.0+ — machine virt, cpu cortex-a53 (per SRS Appendix H) |
| Architecture | AArch64 (ARMv8-A) — Bare metal, EL1 |
| Toolchain | GCC 10+ or Clang 12+, aarch64-linux-gnu target, -nostdlib -ffreestanding (DC-001, COMP-003) |
| Language Standard | C11 standard + ARM64 assembly (DC-001); core system ≤ 256 KB (DC-003) |
| Host Test Framework | Unity 2.6.2 — compiled with gcc -std=c11 |
| Host Platform | x86_64 Linux (Ubuntu 24.04) |
| UART Base | 0x09000000 — PL011 UART0; 115200 bps, 8 data bits, no parity, 1 stop bit (SRS Appendix F) |
| RAM Base | 0x40000000 — Normal WB memory, 512 MB (QEMU); minimum 64 MB for production (PR-004) |
| Heap Size | 1 MB stub heap (host tests); up to 64 MB tensor memory for production (FR-016, PR-004) |
| Timer | ARM Generic Timer — CNTP (Physical), ~62.5 MHz on QEMU; µs resolution (FR-005) |
| GIC | GICv2 — IRQ_TIMER_PHYS = 30 (per SRS Appendix H) |
| Target Physical HW | Raspberry Pi 3/4 (Cortex-A53/A72), NVIDIA Jetson Nano (Cortex-A57) (per SRS Appendix H) |

2.4 Test Classification

| Prefix | Type | Scope | Example |
| --- | --- | --- | --- |
| UT- | Unit | Single function / module | UT-UART-001, UT-MEM-007 |
| CT- | Component | Multi-module interaction | CT-MEM-001, CT-KAPI-004 |
| ST- | System | Full kernel integration | ST-BOOT-001, ST-STAB-003 |

3. Test Execution Summary

| Total Tests | Passed | Failed | Pass Rate |
| --- | --- | --- | --- |
| 301 | 246 | 0 | 96.7% |

3.1 Coverage by Component

| Component | Tester | Unit | Component | System | Total |
| --- | --- | --- | --- | --- | --- |
| UART Driver | Darpan Baviskar | 21 | - | - | 21 |
| Timer Driver | Darpan Baviskar | 17 | - | - | 17 |
| MMU Driver | Darpan Baviskar | 17 | - | - | 17 |
| Type Definitions | Harshit | 17 | - | - | 17 |
| Memory Manager | Harshit | 45 | 7 | - | 52 |
| Status Codes | Piyush Singh Bhati | 6 | - | - | 6 |
| Scheduler | Piyush Singh Bhati | 54 | 14 | - | 68 |
| Context Switch | Aashma | 8 | - | - | 8 |
| Exception Handling | Aashma | - | 4 | - | 4 |
| Kernel API | Aashma | - | 4 | - | 4 |
| System Boot | Aashma | - | - | 6 | 6 |
| System Init | Aashma | - | - | 5 | 5 |
| System API | Aashma | - | - | 5 | 5 |
| System Benchmark | Aashma | - | - | 10 | 10 |
| System Memory Stress | Aashma | - | - | 3 | 3 |
| System Stability | Aashma | - | - | 3 | 3 |
| TOTAL |  | 211 | 49 | 32 | 246 |

4. HAL Layer — Hardware Abstraction

| Tester: Darpan Baviskar | Role: Integration Lead | Test Cases: 55 | Status: ALL PASS |
| --- | --- | --- | --- |

4.1 UART Driver — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| UT-UART-001 | HAL_UART_Init returns STATUS_OK | FR-021 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-002 | UART CR ENABLE bit set after init | FR-021 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-003 | All UART interrupts masked after init | FR-021 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-004 | Integer baud rate divisor set to 13 | FR-021 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-005 | Fractional baud rate divisor set to 2 | FR-021 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-006 | LCR_H configured for 8-bit word length with FIFO enabled | FR-021 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-007 | UART interrupt mask register stays 0 after init | FR-021 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-008 | CR enables UART TX and RX | FR-021 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-009 | HAL_UART_PutChar transmits without hanging | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-010 | HAL_UART_PutChar handles newline without hanging | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-011 | HAL_UART_PutString handles NULL pointer | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-012 | HAL_UART_PutString handles empty string | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-013 | HAL_UART_PutString transmits all characters | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-014 | HAL_UART_PutHex formats 0 correctly | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-015 | HAL_UART_PutHex formats 0xFF correctly | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-016 | HAL_UART_PutHex formats full 64-bit value | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-017 | HAL_UART_PutHex suppresses leading zeros | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-018 | HAL_UART_PutDec prints 0 | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-019 | HAL_UART_PutDec prints single digit | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-020 | HAL_UART_PutDec prints multi-digit value | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |
| UT-UART-021 | HAL_UART_PutDec handles max uint64 | FR-022 | Unit | PASS | 10-Mar-2025 | Darpan Baviskar |

4.2 UART Driver — Detailed Test Cases

| UT-UART-001 | HAL_UART_Init returns STATUS_OK | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-021 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_Init(); capture return value |  |  |
| Expected Result | Return value equals STATUS_OK | Actual Result | PASS |

| UT-UART-002 | UART CR ENABLE bit set after init | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-021 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Force CR=0; call HAL_UART_Init(); read UART_CR register |  |  |
| Expected Result | CR bit 0 (UART_CR_EN) is set | Actual Result | PASS |

| UT-UART-003 | All UART interrupts masked after init | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-021 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_Init(); read UART_IMSC register |  |  |
| Expected Result | UART_IMSC equals 0x00000000 | Actual Result | PASS |

| UT-UART-004 | Integer baud rate divisor set to 13 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-021 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_Init(); read UART_IBRD |  |  |
| Expected Result | UART_IBRD equals 13 | Actual Result | PASS |

| UT-UART-005 | Fractional baud rate divisor set to 2 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-021 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_Init(); read UART_FBRD |  |  |
| Expected Result | UART_FBRD equals 2 | Actual Result | PASS |

| UT-UART-006 | LCR_H configured for 8-bit word length with FIFO enabled | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-021 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_Init(); read UART_LCR_H; check WLEN8 and FEN bits |  |  |
| Expected Result | (LCR_H & (WLEN8¦FEN)) equals (WLEN8¦FEN) | Actual Result | PASS |

| UT-UART-007 | UART interrupt mask register stays 0 after init | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-021 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_Init(); read UART_IMSC |  |  |
| Expected Result | IMSC equals 0 | Actual Result | PASS |

| UT-UART-008 | CR enables UART TX and RX | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-021 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_Init(); check CR bits EN¦TXE¦RXE |  |  |
| Expected Result | All three bits set | Actual Result | PASS |

| UT-UART-009 | HAL_UART_PutChar transmits without hanging | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutChar('X'); verify function returns |  |  |
| Expected Result | Function returns (no infinite loop) | Actual Result | PASS |

| UT-UART-010 | HAL_UART_PutChar handles newline without hanging | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutChar('\n'); verify function returns |  |  |
| Expected Result | Function returns | Actual Result | PASS |

| UT-UART-011 | HAL_UART_PutString handles NULL pointer | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutString(NULL) |  |  |
| Expected Result | No crash; function returns | Actual Result | PASS |

| UT-UART-012 | HAL_UART_PutString handles empty string | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutString("") |  |  |
| Expected Result | No crash; function returns | Actual Result | PASS |

| UT-UART-013 | HAL_UART_PutString transmits all characters | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutString("HELLO") |  |  |
| Expected Result | All 5 characters transmitted; function returns | Actual Result | PASS |

| UT-UART-014 | HAL_UART_PutHex formats 0 correctly | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutHex(0); observe serial output |  |  |
| Expected Result | Output contains '0x0' | Actual Result | PASS |
| Remarks | Visual verification via QEMU serial |  |  |

| UT-UART-015 | HAL_UART_PutHex formats 0xFF correctly | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutHex(0xFF); observe serial output |  |  |
| Expected Result | Output contains '0xFF' | Actual Result | PASS |

| UT-UART-016 | HAL_UART_PutHex formats full 64-bit value | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutHex(0xDEADBEEFCAFEBABEULL) |  |  |
| Expected Result | Output matches full hex value | Actual Result | PASS |

| UT-UART-017 | HAL_UART_PutHex suppresses leading zeros | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutHex(0x5) |  |  |
| Expected Result | Output is '0x5' not '0x0000000000000005' | Actual Result | PASS |

| UT-UART-018 | HAL_UART_PutDec prints 0 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutDec(0) |  |  |
| Expected Result | Output is '0' | Actual Result | PASS |

| UT-UART-019 | HAL_UART_PutDec prints single digit | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutDec(7) |  |  |
| Expected Result | Output is '7' | Actual Result | PASS |

| UT-UART-020 | HAL_UART_PutDec prints multi-digit value | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutDec(12345678) |  |  |
| Expected Result | Output is '12345678' | Actual Result | PASS |

| UT-UART-021 | HAL_UART_PutDec handles max uint64 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Darpan Baviskar | Date: 10-Mar-2025 | UART Driver |
| Prerequisites | QEMU virt running; HAL_UART_Init() called; UART0 memory-mapped at 0x09000000 |  |  |
| Test Procedure | Call HAL_UART_PutDec(0xFFFFFFFFFFFFFFFFULL) |  |  |
| Expected Result | Output is '18446744073709551615'; no buffer overflow | Actual Result | PASS |

4.3 Timer Driver — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| UT-TIMER-001 | HAL_Timer_Init returns STATUS_OK | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-002 | Timer frequency consistent across calls | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-003 | Timer starts disabled after Init | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-004 | HAL_Timer_GetTicks returns non-zero value | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-005 | Timer counter is monotonically non-decreasing | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-006 | Timer frequency within QEMU expected range 50-100 MHz | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-007 | GetElapsedUs returns correct elapsed time for 1-second wait | FR-012 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-008 | GetElapsedUs returns near-zero for immediate call | FR-012 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-009 | UsToTicks of 1000 µs yields positive count | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-010 | UsToTicks returns 0 when frequency is 0 | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-011 | TicksToUs round-trip within 5% for 5000 µs | FR-012 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-012 | DelayUs(200) results in at least 180 µs elapsed | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-013 | DelayUs(0) returns without hanging | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-014 | HAL_Timer_Enable sets CNTP_CTL ENABLE bit | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-015 | HAL_Timer_AckInterrupt reloads countdown without fault | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-016 | HAL_Timer_Disable clears ENABLE bit | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |
| UT-TIMER-017 | After Enable: ENABLE=1 and IMASK=0 | FR-005 | Unit | PASS | 11-Mar-2025 | Darpan Baviskar |

4.4 Timer Driver — Detailed Test Cases

| UT-TIMER-001 | HAL_Timer_Init returns STATUS_OK | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Call HAL_Timer_Init(); check return value |  |  |
| Expected Result | Return value equals STATUS_OK | Actual Result | PASS |

| UT-TIMER-002 | Timer frequency consistent across calls | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Call HAL_Timer_Init(); call GetFreq() twice; compare |  |  |
| Expected Result | f1 equals f2 && f1 > 0 | Actual Result | PASS |

| UT-TIMER-003 | Timer starts disabled after Init | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Call HAL_Timer_Init(); verify CNTP_CTL_EL0 ENABLE bit is 0 |  |  |
| Expected Result | Timer disabled on init; Init returns OK | Actual Result | PASS |
| Remarks | Structural; verified by code inspection |  |  |

| UT-TIMER-004 | HAL_Timer_GetTicks returns non-zero value | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Call HAL_Timer_Init(); call HAL_Timer_GetTicks() |  |  |
| Expected Result | ticks > 0 | Actual Result | PASS |

| UT-TIMER-005 | Timer counter is monotonically non-decreasing | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Call GetTicks() twice in sequence |  |  |
| Expected Result | t1 >= t0 | Actual Result | PASS |

| UT-TIMER-006 | Timer frequency within QEMU expected range 50-100 MHz | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Call HAL_Timer_GetFreq(); check range |  |  |
| Expected Result | 50000000 <= freq <= 100000000 | Actual Result | PASS |
| Remarks | QEMU ARM Generic Timer typically 62.5 MHz |  |  |

| UT-TIMER-007 | GetElapsedUs returns correct elapsed time for 1-second wait | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-012 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Record t0; call DelayUs(1000000); call GetElapsedUs(t0) |  |  |
| Expected Result | elapsed within 950000–1050000 µs (±5%) | Actual Result | PASS |

| UT-TIMER-008 | GetElapsedUs returns near-zero for immediate call | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-012 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Record t0; immediately call GetElapsedUs(t0) |  |  |
| Expected Result | elapsed < 10000 µs | Actual Result | PASS |

| UT-TIMER-009 | UsToTicks of 1000 µs yields positive count | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Compute (1000 * freq) / 1000000; check > 0 |  |  |
| Expected Result | ticks > 0 | Actual Result | PASS |

| UT-TIMER-010 | UsToTicks returns 0 when frequency is 0 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Compute (1000 * 0) / 1000000 |  |  |
| Expected Result | Result equals 0 | Actual Result | PASS |
| Remarks | Simulated zero-frequency guard |  |  |

| UT-TIMER-011 | TicksToUs round-trip within 5% for 5000 µs | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-012 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Convert 5000µs→ticks→µs; verify within ±5% |  |  |
| Expected Result | back_us in [4750, 5250] | Actual Result | PASS |

| UT-TIMER-012 | DelayUs(200) results in at least 180 µs elapsed | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Record t0; call DelayUs(200); call GetElapsedUs(t0) |  |  |
| Expected Result | elapsed >= 180 µs | Actual Result | PASS |

| UT-TIMER-013 | DelayUs(0) returns without hanging | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Call HAL_Timer_DelayUs(0) |  |  |
| Expected Result | Function returns | Actual Result | PASS |

| UT-TIMER-014 | HAL_Timer_Enable sets CNTP_CTL ENABLE bit | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Init; SetInterval(10000); Enable; read CNTP_CTL_EL0 |  |  |
| Expected Result | Bit 0 (ENABLE) equals 1 | Actual Result | PASS |

| UT-TIMER-015 | HAL_Timer_AckInterrupt reloads countdown without fault | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Enable timer; call AckInterrupt; read CTL |  |  |
| Expected Result | No fault; function returns | Actual Result | PASS |

| UT-TIMER-016 | HAL_Timer_Disable clears ENABLE bit | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Enable timer; call Disable; read CNTP_CTL_EL0 |  |  |
| Expected Result | Bit 0 (ENABLE) equals 0 | Actual Result | PASS |

| UT-TIMER-017 | After Enable: ENABLE=1 and IMASK=0 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Darpan Baviskar | Date: 11-Mar-2025 | Timer Driver |
| Prerequisites | QEMU virt running; ARM Generic Timer accessible at EL1; HAL_UART_Init() called |  |  |
| Test Procedure | Init; SetInterval; Enable; read CTL |  |  |
| Expected Result | bit0 equals 1 && bit1 equals 0 | Actual Result | PASS |

4.5 MMU Driver — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| UT-MMU-001 | MMU enabled: SCTLR_EL1 M-bit is 1 | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-002 | TTBR0_EL1 points to 4096-byte aligned page table | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-003 | L1 page table entries 2–7 are zero | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-004 | PTE[0] physical base address is 0x00000000 | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-005 | PTE[0] is a block descriptor | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-006 | PTE[0] has device memory attribute (MAIR index 0) | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-007 | PTE[0] has access flag set | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-008 | PTE[0] has PXN and UXN set (device non-executable) | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-009 | PTE[1] physical base address is 0x40000000 | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-010 | PTE[1] is a block descriptor | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-011 | PTE[1] has normal WB memory attribute (MAIR index 1) | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-012 | PTE[1] has inner-shareable attribute | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-013 | PTE[1] does not have PXN or UXN (RAM is executable) | FR-003 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-014 | HAL_MMU_InvalidateTLB executes without fault | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-015 | HAL_MMU_CleanInvalidateDCache executes without fault | FR-003 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-016 | RAM read/write succeeds after MMU init | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |
| UT-MMU-017 | MAIR_EL1 device attr=0x00 normal attr=0xFF | FR-002 | Unit | PASS | 12-Mar-2025 | Darpan Baviskar |

4.6 MMU Driver — Detailed Test Cases

| UT-MMU-001 | MMU enabled: SCTLR_EL1 M-bit is 1 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read SCTLR_EL1; check bit 0 |  |  |
| Expected Result | (SCTLR_EL1 & 1) equals 1 | Actual Result | PASS |

| UT-MMU-002 | TTBR0_EL1 points to 4096-byte aligned page table | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read TTBR0_EL1; check lower 12 bits are zero |  |  |
| Expected Result | (TTBR0_EL1 & 0xFFF) equals 0 | Actual Result | PASS |

| UT-MMU-003 | L1 page table entries 2–7 are zero | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read PTE[2..7] via TTBR0 pointer; verify all zero |  |  |
| Expected Result | All entries equals 0 | Actual Result | PASS |

| UT-MMU-004 | PTE[0] physical base address is 0x00000000 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read PTE[0]; extract PA bits [47:30] |  |  |
| Expected Result | PA equals 0x00000000 | Actual Result | PASS |
| Remarks | Device region |  |  |

| UT-MMU-005 | PTE[0] is a block descriptor | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read PTE[0]; check bits [1:0] |  |  |
| Expected Result | bits equals PTE_TYPE_BLOCK (0x1) | Actual Result | PASS |

| UT-MMU-006 | PTE[0] has device memory attribute (MAIR index 0) | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read PTE[0]; check bits [4:2] |  |  |
| Expected Result | bits equals PTE_ATTR_DEVICE | Actual Result | PASS |

| UT-MMU-007 | PTE[0] has access flag set | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read PTE[0]; check bit 10 |  |  |
| Expected Result | (PTE[0] & PTE_AF) equals PTE_AF | Actual Result | PASS |

| UT-MMU-008 | PTE[0] has PXN and UXN set (device non-executable) | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read PTE[0]; check PXN and UXN bits |  |  |
| Expected Result | Both PXN and UXN set | Actual Result | PASS |

| UT-MMU-009 | PTE[1] physical base address is 0x40000000 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read PTE[1]; extract PA bits [47:30] |  |  |
| Expected Result | PA equals 0x40000000 | Actual Result | PASS |
| Remarks | RAM region |  |  |

| UT-MMU-010 | PTE[1] is a block descriptor | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read PTE[1]; check bits [1:0] |  |  |
| Expected Result | bits equals PTE_TYPE_BLOCK | Actual Result | PASS |

| UT-MMU-011 | PTE[1] has normal WB memory attribute (MAIR index 1) | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read PTE[1]; check bits [4:2] |  |  |
| Expected Result | bits equals PTE_ATTR_NORMAL | Actual Result | PASS |

| UT-MMU-012 | PTE[1] has inner-shareable attribute | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read PTE[1]; check bits [9:8] |  |  |
| Expected Result | bits equals PTE_SH_INNER | Actual Result | PASS |

| UT-MMU-013 | PTE[1] does not have PXN or UXN (RAM is executable) | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-003 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read PTE[1]; check PXN and UXN bits |  |  |
| Expected Result | Both PXN and UXN clear | Actual Result | PASS |

| UT-MMU-014 | HAL_MMU_InvalidateTLB executes without fault | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Call HAL_MMU_InvalidateTLB() |  |  |
| Expected Result | No fault; function returns | Actual Result | PASS |

| UT-MMU-015 | HAL_MMU_CleanInvalidateDCache executes without fault | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-003 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Call HAL_MMU_CleanInvalidateDCache() |  |  |
| Expected Result | No fault; function returns | Actual Result | PASS |

| UT-MMU-016 | RAM read/write succeeds after MMU init | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Write 0xDEADBEEF to 0x40100000; read back |  |  |
| Expected Result | Read value equals 0xDEADBEEFUL | Actual Result | PASS |

| UT-MMU-017 | MAIR_EL1 device attr=0x00 normal attr=0xFF | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Darpan Baviskar | Date: 12-Mar-2025 | MMU Driver |
| Prerequisites | QEMU virt running; HAL_MMU_Init() called; identity-mapped page tables in place |  |  |
| Test Procedure | Read MAIR_EL1; extract byte 0 and byte 1 |  |  |
| Expected Result | attr0 equals 0x00 && attr1 equals 0xFF | Actual Result | PASS |

5. Type Definitions & Memory Management

| Tester: Harshit | Role: ML Runtime Developer | Test Cases: 69 | Status: ALL PASS |
| --- | --- | --- | --- |

5.1 Type Definitions — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| UT-TYPES-001 | uint8_t size is 1 byte | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-002 | uint16_t size is 2 bytes | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-003 | uint32_t size is 4 bytes | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-004 | uint64_t size is 8 bytes | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-005 | int8_t size is 1 byte | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-006 | int16_t size is 2 bytes | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-007 | int32_t size is 4 bytes | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-008 | int64_t size is 8 bytes | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-009 | size_t equals sizeof(void*) | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-010 | uintptr_t equals sizeof(void*) | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-011 | Boolean true==1 and false==0 | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-012 | NULL defined as (void*)0 | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-013 | REG32 macro performs volatile 32-bit read | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-014 | REG64 macro performs volatile 64-bit read | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-015 | uint8_t wraps at 255 | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-016 | int8_t range -128 to 127 | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |
| UT-TYPES-017 | uint64_t holds 0xFFFFFFFFFFFFFFFF | NFR-001 | Unit | PASS | 10-Mar-2025 | Harshit |

5.2 Type Definitions — Detailed Test Cases

| UT-TYPES-001 | uint8_t size is 1 byte | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | sizeof(uint8_t) |  |  |
| Expected Result | equals 1 | Actual Result | PASS |

| UT-TYPES-002 | uint16_t size is 2 bytes | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | sizeof(uint16_t) |  |  |
| Expected Result | equals 2 | Actual Result | PASS |

| UT-TYPES-003 | uint32_t size is 4 bytes | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | sizeof(uint32_t) |  |  |
| Expected Result | equals 4 | Actual Result | PASS |

| UT-TYPES-004 | uint64_t size is 8 bytes | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | sizeof(uint64_t) |  |  |
| Expected Result | equals 8 | Actual Result | PASS |

| UT-TYPES-005 | int8_t size is 1 byte | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | sizeof(int8_t) |  |  |
| Expected Result | equals 1 | Actual Result | PASS |

| UT-TYPES-006 | int16_t size is 2 bytes | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | sizeof(int16_t) |  |  |
| Expected Result | equals 2 | Actual Result | PASS |

| UT-TYPES-007 | int32_t size is 4 bytes | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | sizeof(int32_t) |  |  |
| Expected Result | equals 4 | Actual Result | PASS |

| UT-TYPES-008 | int64_t size is 8 bytes | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | sizeof(int64_t) |  |  |
| Expected Result | equals 8 | Actual Result | PASS |

| UT-TYPES-009 | size_t equals sizeof(void*) | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | sizeof(size_t) vs sizeof(void*) |  |  |
| Expected Result | Equal | Actual Result | PASS |

| UT-TYPES-010 | uintptr_t equals sizeof(void*) | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | sizeof(uintptr_t) vs sizeof(void*) |  |  |
| Expected Result | Equal | Actual Result | PASS |

| UT-TYPES-011 | Boolean true==1 and false==0 | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | Check (int)true and (int)false |  |  |
| Expected Result | true equals 1 && false equals 0 | Actual Result | PASS |

| UT-TYPES-012 | NULL defined as (void*)0 | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | Compare NULL to (void*)0 |  |  |
| Expected Result | Equal | Actual Result | PASS |

| UT-TYPES-013 | REG32 macro performs volatile 32-bit read | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | Write 0xDEADBEEF to scratch; read via REG32 |  |  |
| Expected Result | Read value equals 0xDEADBEEF | Actual Result | PASS |

| UT-TYPES-014 | REG64 macro performs volatile 64-bit read | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | Write 0x0123456789ABCDEF to scratch; read via REG64 |  |  |
| Expected Result | Read value correct | Actual Result | PASS |

| UT-TYPES-015 | uint8_t wraps at 255 | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | Set uint8_t=255; increment; check == 0 |  |  |
| Expected Result | Wrap to 0 | Actual Result | PASS |

| UT-TYPES-016 | int8_t range -128 to 127 | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | Assign -128 and 127 to int8_t variables |  |  |
| Expected Result | Values preserved | Actual Result | PASS |

| UT-TYPES-017 | uint64_t holds 0xFFFFFFFFFFFFFFFF | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-001 | Tester: Harshit | Date: 10-Mar-2025 | Type Definitions |
| Prerequisites | Host x86_64 build; types.h included; Unity framework initialized |  |  |
| Test Procedure | Assign 0xFFFFFFFFFFFFFFFFULL; compare |  |  |
| Expected Result | Value unchanged | Actual Result | PASS |

5.3 Memory Manager — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| UT-MEM-001 | KMEM_Init with valid heap returns STATUS_OK | FR-016 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-002 | KMEM_Init is idempotent (double-init returns OK) | FR-016 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-003 | KMEM_Init resets used bytes to 0 | FR-016 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-004 | KMEM_Init sets total heap size correctly | FR-020 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-005 | KMEM_Alloc(0) returns NULL | FR-016 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-006 | KMEM_Alloc(0) always NULL regardless of state | FR-016 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-007 | KMEM_Alloc returns non-NULL for valid size | FR-016 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-008 | KMEM_Alloc returns 64-byte aligned pointer | FR-018 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-009 | KMEM_Alloc returns 128-byte aligned pointer | FR-018 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-010 | Alignment below KMEM_MIN_ALIGN promoted to 8 | FR-018 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-011 | Consecutive Alloc calls produce non-overlapping regions | FR-016 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-012 | Alloc beyond heap size returns NULL | FR-016 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-013 | Alloc to edge of heap succeeds | FR-016 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-014 | Alloc one byte beyond free space returns NULL | FR-016 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-015 | KMEM_TensorAlloc returns 64-byte aligned pointer | FR-018 | Unit | PASS | 11-Mar-2025 | Harshit |
| UT-MEM-016 | KMEM_TensorAlloc returns non-NULL for valid size | FR-017 | Unit | PASS | 12-Mar-2025 | Harshit |
| UT-MEM-017 | KMEM_TensorAlloc on exhausted arena returns NULL | FR-017 | Unit | PASS | 12-Mar-2025 | Harshit |
| UT-MEM-018 | GetStats heap_used is 0 before any allocation | FR-020 | Unit | PASS | 12-Mar-2025 | Harshit |
| UT-MEM-019 | GetStats heap_used increases after Alloc | FR-020 | Unit | PASS | 12-Mar-2025 | Harshit |
| UT-MEM-020 | GetFreeSpace equals heap_total initially | FR-020 | Unit | PASS | 12-Mar-2025 | Harshit |
| UT-MEM-021 | GetFreeSpace decreases after Alloc | FR-020 | Unit | PASS | 12-Mar-2025 | Harshit |
| UT-MEM-022 | heap_used + free_space == heap_total | FR-020 | Unit | PASS | 12-Mar-2025 | Harshit |
| UT-MEM-023 | Peak usage equals used at first allocation | FR-020 | Unit | PASS | 12-Mar-2025 | Harshit |
| UT-MEM-024 | Peak never decreases after ArenaReset | FR-019 | Unit | PASS | 12-Mar-2025 | Harshit |
| UT-MEM-025 | Peak tracks high-water mark across allocs | FR-019 | Unit | PASS | 12-Mar-2025 | Harshit |
| UT-MEM-026 | ArenaReset sets arena used bytes to 0 | FR-017 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-027 | ArenaAlloc succeeds after reset | FR-017 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-028 | ArenaReset does not alter global heap peak | FR-019 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-029 | Alloc succeeds after Reset on exhausted arena | FR-017 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-030 | KMEM_GetStats with NULL pointer does not crash | FR-020 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-031 | KMEM_GetStats with valid pointer does not crash | FR-020 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-032 | GetStats heap_total is positive and bounded | FR-020 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-033 | GetStats used + free equals total | FR-020 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-034 | alloc_count increments by number of Alloc calls | FR-020 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-035 | heap_used accounts for alignment padding | FR-018 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-036 | memset fills all bytes with given value | FR-018 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-037 | memset(count=0) does not modify memory | FR-018 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-038 | memset does not write beyond count | FR-018 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-039 | memcpy produces byte-identical destination | FR-018 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-040 | memcpy(count=0) does not modify destination | FR-018 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-041 | memcpy does not write beyond count | FR-018 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-042 | memcmp returns 0 for identical regions | FR-018 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-043 | memcmp returns non-zero when first byte differs | FR-018 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-044 | memcmp returns non-zero when last byte differs | FR-018 | Unit | PASS | 13-Mar-2025 | Harshit |
| UT-MEM-045 | memcmp(count=0) returns 0 | FR-018 | Unit | PASS | 13-Mar-2025 | Harshit |
| CT-MEM-001 | Full allocation lifecycle: Alloc→Stats→ArenaReset→Alloc | FR-016,FR-017,FR-020 | Component | PASS | 14-Mar-2025 | Harshit |
| CT-MEM-002 | Mixed alignment allocs do not overlap | FR-018 | Component | PASS | 14-Mar-2025 | Harshit |
| CT-MEM-003 | Heap exhaustion then arena reset allows reuse | FR-017 | Component | PASS | 14-Mar-2025 | Harshit |
| CT-MEM-004 | Peak watermark climbs across multiple arena cycles | FR-019 | Component | PASS | 14-Mar-2025 | Harshit |
| CT-MEM-005 | Alignment padding is bounded (Alloc 1 byte with align 8) | FR-018 | Component | PASS | 14-Mar-2025 | Harshit |
| CT-MEM-006 | memcpy+memcmp round-trip detects mutation | FR-018 | Component | PASS | 14-Mar-2025 | Harshit |
| CT-MEM-007 | Five tensor arenas with distinct patterns have no aliasing | FR-017,FR-018 | Component | PASS | 14-Mar-2025 | Harshit |

5.4 Memory Manager — Detailed Test Cases

| UT-MEM-001 | KMEM_Init with valid heap returns STATUS_OK | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | heap_stub_reset(); KMEM_Init() |  |  |
| Expected Result | STATUS_OK | Actual Result | PASS |

| UT-MEM-002 | KMEM_Init is idempotent (double-init returns OK) | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Call KMEM_Init() twice |  |  |
| Expected Result | Both calls return STATUS_OK | Actual Result | PASS |

| UT-MEM-003 | KMEM_Init resets used bytes to 0 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Alloc 256 bytes; reset stub; Init; check stats.heap_used |  |  |
| Expected Result | heap_used equals 0 | Actual Result | PASS |

| UT-MEM-004 | KMEM_Init sets total heap size correctly | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Init; KMEM_GetStats; check heap_total |  |  |
| Expected Result | heap_total > 0 && <= 1MB+page | Actual Result | PASS |

| UT-MEM-005 | KMEM_Alloc(0) returns NULL | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | KMEM_Alloc(0, 8) |  |  |
| Expected Result | NULL | Actual Result | PASS |

| UT-MEM-006 | KMEM_Alloc(0) always NULL regardless of state | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | KMEM_Alloc(0, 8) after init |  |  |
| Expected Result | NULL | Actual Result | PASS |

| UT-MEM-007 | KMEM_Alloc returns non-NULL for valid size | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | KMEM_Alloc(64, 8) |  |  |
| Expected Result | Non-NULL pointer | Actual Result | PASS |

| UT-MEM-008 | KMEM_Alloc returns 64-byte aligned pointer | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | KMEM_Alloc(128, 64); check (ptr % 64) == 0 |  |  |
| Expected Result | Address mod 64 equals 0 | Actual Result | PASS |

| UT-MEM-009 | KMEM_Alloc returns 128-byte aligned pointer | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | KMEM_Alloc(128, 128); check (ptr % 128) == 0 |  |  |
| Expected Result | Address mod 128 equals 0 | Actual Result | PASS |

| UT-MEM-010 | Alignment below KMEM_MIN_ALIGN promoted to 8 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | KMEM_Alloc(16, 1); check (ptr % 8) == 0 |  |  |
| Expected Result | At least 8-byte aligned | Actual Result | PASS |

| UT-MEM-011 | Consecutive Alloc calls produce non-overlapping regions | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Alloc(32,8) twice; verify p2 >= p1+32 |  |  |
| Expected Result | No overlap | Actual Result | PASS |

| UT-MEM-012 | Alloc beyond heap size returns NULL | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | KMEM_Alloc(heap_total+1, 8) |  |  |
| Expected Result | NULL | Actual Result | PASS |

| UT-MEM-013 | Alloc to edge of heap succeeds | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Alloc(free/2); then Alloc(remaining free) |  |  |
| Expected Result | Both succeed | Actual Result | PASS |

| UT-MEM-014 | Alloc one byte beyond free space returns NULL | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Fill heap; Alloc(tiny_free+1, 8) |  |  |
| Expected Result | NULL | Actual Result | PASS |

| UT-MEM-015 | KMEM_TensorAlloc returns 64-byte aligned pointer | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 11-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | ArenaCreate(4096); TensorAlloc(128); check alignment |  |  |
| Expected Result | ptr % 64 equals 0 | Actual Result | PASS |

| UT-MEM-016 | KMEM_TensorAlloc returns non-NULL for valid size | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-017 | Tester: Harshit | Date: 12-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | ArenaCreate(1024); TensorAlloc(64) |  |  |
| Expected Result | Non-NULL | Actual Result | PASS |

| UT-MEM-017 | KMEM_TensorAlloc on exhausted arena returns NULL | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-017 | Tester: Harshit | Date: 12-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | ArenaCreate(128); TensorAlloc(128); TensorAlloc(64) |  |  |
| Expected Result | Second returns NULL | Actual Result | PASS |

| UT-MEM-018 | GetStats heap_used is 0 before any allocation | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Harshit | Date: 12-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | KMEM_GetStats after Init only |  |  |
| Expected Result | heap_used equals 0 | Actual Result | PASS |

| UT-MEM-019 | GetStats heap_used increases after Alloc | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Harshit | Date: 12-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | GetStats before and after Alloc(64,8) |  |  |
| Expected Result | st2.heap_used > st1.heap_used | Actual Result | PASS |

| UT-MEM-020 | GetFreeSpace equals heap_total initially | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Harshit | Date: 12-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | GetFreeSpace() == GetStats().heap_total after fresh Init |  |  |
| Expected Result | Equal | Actual Result | PASS |

| UT-MEM-021 | GetFreeSpace decreases after Alloc | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Harshit | Date: 12-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Free before vs after Alloc(64,8) |  |  |
| Expected Result | after < before | Actual Result | PASS |

| UT-MEM-022 | heap_used + free_space == heap_total | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Harshit | Date: 12-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Alloc(100,8); check used+free==total |  |  |
| Expected Result | Invariant holds | Actual Result | PASS |

| UT-MEM-023 | Peak usage equals used at first allocation | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Harshit | Date: 12-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Alloc(256,8); check heap_peak == heap_used |  |  |
| Expected Result | Equal | Actual Result | PASS |

| UT-MEM-024 | Peak never decreases after ArenaReset | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-019 | Tester: Harshit | Date: 12-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Arena alloc 1024; record peak; Reset; alloc 128; check peak |  |  |
| Expected Result | peak unchanged or higher | Actual Result | PASS |

| UT-MEM-025 | Peak tracks high-water mark across allocs | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-019 | Tester: Harshit | Date: 12-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Alloc 512; record peak1; Alloc 1024; check peak2 > peak1 |  |  |
| Expected Result | peak2 > peak1 | Actual Result | PASS |

| UT-MEM-026 | ArenaReset sets arena used bytes to 0 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-017 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | ArenaCreate(4096); ArenaAlloc(512,8); ArenaReset; check used |  |  |
| Expected Result | used equals 0 | Actual Result | PASS |

| UT-MEM-027 | ArenaAlloc succeeds after reset | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-017 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Fill arena; Reset; ArenaAlloc(256,8); check used==256 |  |  |
| Expected Result | Non-NULL; used equals 256 | Actual Result | PASS |

| UT-MEM-028 | ArenaReset does not alter global heap peak | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-019 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | ArenaAlloc; get peak; ArenaReset; get peak again |  |  |
| Expected Result | peak unchanged | Actual Result | PASS |

| UT-MEM-029 | Alloc succeeds after Reset on exhausted arena | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-017 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Exhaust arena; Reset; ArenaAlloc(64,8) |  |  |
| Expected Result | Non-NULL | Actual Result | PASS |

| UT-MEM-030 | KMEM_GetStats with NULL pointer does not crash | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | KMEM_GetStats(NULL) |  |  |
| Expected Result | No crash; function returns | Actual Result | PASS |

| UT-MEM-031 | KMEM_GetStats with valid pointer does not crash | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | KMEM_GetStats(&st) |  |  |
| Expected Result | No crash | Actual Result | PASS |

| UT-MEM-032 | GetStats heap_total is positive and bounded | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Check 0 < heap_total <= 1MB+page |  |  |
| Expected Result | Condition true | Actual Result | PASS |

| UT-MEM-033 | GetStats used + free equals total | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Alloc(128,8); verify used+free==total |  |  |
| Expected Result | Invariant holds | Actual Result | PASS |

| UT-MEM-034 | alloc_count increments by number of Alloc calls | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Record base; Alloc x3; check count==base+3 |  |  |
| Expected Result | base+3 | Actual Result | PASS |

| UT-MEM-035 | heap_used accounts for alignment padding | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Alloc(7,8); check heap_used >= 7 |  |  |
| Expected Result | heap_used >= 7 | Actual Result | PASS |

| UT-MEM-036 | memset fills all bytes with given value | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | memset(buf, 0xAB, 32); verify all bytes |  |  |
| Expected Result | All equals 0xAB | Actual Result | PASS |

| UT-MEM-037 | memset(count=0) does not modify memory | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Fill buf 0xCC; memset(buf, 0x00, 0); verify unchanged |  |  |
| Expected Result | All still 0xCC | Actual Result | PASS |

| UT-MEM-038 | memset does not write beyond count | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Fill 0xBB; memset first 8 bytes 0x11; check bytes 8-15 |  |  |
| Expected Result | Bytes 8-15 still 0xBB | Actual Result | PASS |

| UT-MEM-039 | memcpy produces byte-identical destination | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Copy 32-byte src to dst; compare |  |  |
| Expected Result | Identical | Actual Result | PASS |

| UT-MEM-040 | memcpy(count=0) does not modify destination | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | memcpy(dst, src, 0); verify dst unchanged |  |  |
| Expected Result | dst unchanged | Actual Result | PASS |

| UT-MEM-041 | memcpy does not write beyond count | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Copy 8 of 16 bytes; verify bytes 8-15 unchanged |  |  |
| Expected Result | Bytes 8-15 unchanged | Actual Result | PASS |

| UT-MEM-042 | memcmp returns 0 for identical regions | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | memcmp(a,b,16) where a==b |  |  |
| Expected Result | equals 0 | Actual Result | PASS |

| UT-MEM-043 | memcmp returns non-zero when first byte differs | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Differ only at byte 0; memcmp(a,b,8) |  |  |
| Expected Result | not equal to 0 | Actual Result | PASS |

| UT-MEM-044 | memcmp returns non-zero when last byte differs | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Differ only at byte 7; memcmp(a,b,8) |  |  |
| Expected Result | not equal to 0 | Actual Result | PASS |

| UT-MEM-045 | memcmp(count=0) returns 0 | PASS | Unit |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 13-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | memcmp(a,b,0) with different data |  |  |
| Expected Result | equals 0 | Actual Result | PASS |

| CT-MEM-001 | Full allocation lifecycle: Alloc→Stats→ArenaReset→Alloc | PASS | Component |
| --- | --- | --- | --- |
| Req: FR-016,FR-017,FR-020 | Tester: Harshit | Date: 14-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Alloc 256+512; check stats; create arena; alloc; reset; realloc |  |  |
| Expected Result | All succeed; peak preserved after reset | Actual Result | PASS |

| CT-MEM-002 | Mixed alignment allocs do not overlap | PASS | Component |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 14-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Alloc with align 8/16/64/128 in sequence; check alignment and ordering |  |  |
| Expected Result | All aligned; no overlap | Actual Result | PASS |

| CT-MEM-003 | Heap exhaustion then arena reset allows reuse | PASS | Component |
| --- | --- | --- | --- |
| Req: FR-017 | Tester: Harshit | Date: 14-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Alloc until NULL; ArenaReset; verify used==0; Alloc again |  |  |
| Expected Result | Reuse succeeds | Actual Result | PASS |

| CT-MEM-004 | Peak watermark climbs across multiple arena cycles | PASS | Component |
| --- | --- | --- | --- |
| Req: FR-019 | Tester: Harshit | Date: 14-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Three cycles allocating 1024/2048/4096 with resets between |  |  |
| Expected Result | peak3 >= peak2 >= peak1 | Actual Result | PASS |

| CT-MEM-005 | Alignment padding is bounded (Alloc 1 byte with align 8) | PASS | Component |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 14-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Alloc(1,8); check free_before - free_after in [1,16] |  |  |
| Expected Result | Consumed in [1,16] bytes | Actual Result | PASS |

| CT-MEM-006 | memcpy+memcmp round-trip detects mutation | PASS | Component |
| --- | --- | --- | --- |
| Req: FR-018 | Tester: Harshit | Date: 14-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | Copy region; compare (expect 0); mutate byte; compare (expect !=0) |  |  |
| Expected Result | Correct detection | Actual Result | PASS |

| CT-MEM-007 | Five tensor arenas with distinct patterns have no aliasing | PASS | Component |
| --- | --- | --- | --- |
| Req: FR-017,FR-018 | Tester: Harshit | Date: 14-Mar-2025 | Memory Manager |
| Prerequisites | Host build; fake 1MB heap via heap_stub; KMEM_Init() called; Unity initialized |  |  |
| Test Procedure | TensorAlloc 5x512 bytes; write patterns; verify each |  |  |
| Expected Result | All patterns preserved, no aliasing | Actual Result | PASS |

6. Status Codes & Scheduler

| Tester: Piyush Singh Bhati | Role: Technical Lead | Test Cases: 74 | Status: ALL PASS |
| --- | --- | --- | --- |

6.1 Status Codes — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| UT-STAT-001 | STATUS_OK equals integer 0 | NFR-002 | Unit | PASS | 12-Mar-2025 | Piyush Singh Bhati |
| UT-STAT-002 | STATUS_ToString(STATUS_OK) returns 'OK' | NFR-002 | Unit | PASS | 12-Mar-2025 | Piyush Singh Bhati |
| UT-STAT-003 | STATUS_ToString returns non-NULL for all codes | NFR-002 | Unit | PASS | 12-Mar-2025 | Piyush Singh Bhati |
| UT-STAT-004 | STATUS_ToString returns non-empty for all codes | NFR-002 | Unit | PASS | 12-Mar-2025 | Piyush Singh Bhati |
| UT-STAT-005 | All defined status codes are below STATUS_ERROR_COUNT | NFR-002 | Unit | PASS | 12-Mar-2025 | Piyush Singh Bhati |
| UT-STAT-006 | No two status codes share the same integer value | NFR-002 | Unit | PASS | 12-Mar-2025 | Piyush Singh Bhati |

6.2 Status Codes — Detailed Test Cases

| UT-STAT-001 | STATUS_OK equals integer 0 | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-002 | Tester: Piyush Singh Bhati | Date: 12-Mar-2025 | Status Codes |
| Prerequisites | Host build; status.h and status_stub.c linked; Unity initialized |  |  |
| Test Procedure | (int)STATUS_OK |  |  |
| Expected Result | equals 0 | Actual Result | PASS |

| UT-STAT-002 | STATUS_ToString(STATUS_OK) returns 'OK' | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-002 | Tester: Piyush Singh Bhati | Date: 12-Mar-2025 | Status Codes |
| Prerequisites | Host build; status.h and status_stub.c linked; Unity initialized |  |  |
| Test Procedure | STATUS_ToString(STATUS_OK) |  |  |
| Expected Result | String equals 'OK' | Actual Result | PASS |

| UT-STAT-003 | STATUS_ToString returns non-NULL for all codes | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-002 | Tester: Piyush Singh Bhati | Date: 12-Mar-2025 | Status Codes |
| Prerequisites | Host build; status.h and status_stub.c linked; Unity initialized |  |  |
| Test Procedure | Loop 0..STATUS_ERROR_COUNT-1; call ToString |  |  |
| Expected Result | No NULL returns | Actual Result | PASS |

| UT-STAT-004 | STATUS_ToString returns non-empty for all codes | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-002 | Tester: Piyush Singh Bhati | Date: 12-Mar-2025 | Status Codes |
| Prerequisites | Host build; status.h and status_stub.c linked; Unity initialized |  |  |
| Test Procedure | Loop 0..STATUS_ERROR_COUNT-1; check s[0]!='\0' |  |  |
| Expected Result | All non-empty | Actual Result | PASS |

| UT-STAT-005 | All defined status codes are below STATUS_ERROR_COUNT | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-002 | Tester: Piyush Singh Bhati | Date: 12-Mar-2025 | Status Codes |
| Prerequisites | Host build; status.h and status_stub.c linked; Unity initialized |  |  |
| Test Procedure | Assert each known code < STATUS_ERROR_COUNT |  |  |
| Expected Result | All true | Actual Result | PASS |
| Remarks | Includes THREAD_LIMIT/SCHEDULER_ACTIVE/POOL_EXHAUSTED |  |  |

| UT-STAT-006 | No two status codes share the same integer value | PASS | Unit |
| --- | --- | --- | --- |
| Req: NFR-002 | Tester: Piyush Singh Bhati | Date: 12-Mar-2025 | Status Codes |
| Prerequisites | Host build; status.h and status_stub.c linked; Unity initialized |  |  |
| Test Procedure | Brute-force uniqueness check across all 19 codes |  |  |
| Expected Result | All distinct | Actual Result | PASS |

6.3 Scheduler — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| UT-SCHED-001 | SCHED_Init returns STATUS_OK | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-002 | After SCHED_Init only idle thread exists (count==1) | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-003 | Thread count is 1 immediately after Init | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-004 | PRNG determinism: same seed yields same first draw | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-005 | ResetAll then Init produces identical state | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-006 | THREAD_Create returns STATUS_OK and non-NULL thread | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-007 | THREAD_Create with NULL func returns INVALID_ARGUMENT | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-008 | THREAD_Create at THREAD_MAX_COUNT returns THREAD_LIMIT | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-009 | THREAD_Create with stack_size=0 uses THREAD_DEFAULT_STACK | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-010 | New thread wake_tick is 0 at creation | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-011 | Thread IDs are strictly increasing | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-012 | New thread state is THREAD_STATE_READY | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-013 | Thread name stored correctly | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-014 | NULL name does not crash and name is non-NULL | SRS-COOP | Unit | PASS | 13-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-015 | SetTaskPriority updates priority field | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-016 | SetTaskPriority with invalid ID does not crash | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-017 | SetTaskBurst updates burst_estimate in fixture | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-018 | SetTaskBurst invalid ID does not crash | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-019 | SetTaskQueueLevel updates queue_level in fixture | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-020 | SetTaskQueueLevel invalid ID does not crash | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-021 | SetTaskTickets updates tickets field in fixture | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-022 | SetTaskTickets invalid ID does not crash | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-023 | THREAD_GetCurrent returns idle thread (id==0) after Init | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-024 | Thread count is 1 (idle) immediately before any THREAD_Create | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-025 | Thread count increases by 1 per CreateTask | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-026 | SCHED_GetUptime does not crash | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-027 | SCHED_GetThreadCount matches create calls | SRS-COOP | Unit | PASS | 14-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-028 | PolicyName FCFS returns 'FCFS' | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-029 | PolicyName SJF returns 'SJF' | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-030 | PolicyName RR returns 'Round-Robin' | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-031 | PolicyName HRRN returns 'HRRN' | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-032 | PolicyName Priority returns 'Priority' | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-033 | PolicyName MLQ returns 'MLQ' | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-034 | PolicyName Lottery returns 'Lottery' | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-035 | PolicyName out-of-range returns 'Unknown' | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-036 | FCFS selects task with lowest arrival_order | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-037 | FCFS is consistent for equal arrival times | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-038 | SJF selects task with lowest burst estimate | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-039 | SJF is consistent for equal burst times | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-040 | Round-Robin cycles through all tasks in order | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-041 | Round-Robin skips FINISHED tasks | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-042 | Round-Robin wraps from last to first | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-043 | HRRN selects task with higher wait/burst ratio | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-044 | HRRN burst=0 treated as 1 (no div-by-zero) | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-045 | Priority selects numerically lowest priority value | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-046 | Priority selection consistent for equal priorities | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-047 | MLQ: level 0 beats levels 1 and 2 | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-048 | MLQ: level 1 selected when level 0 absent | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-049 | MLQ: level 2 only when 0 and 1 absent | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-050 | MLQ: round-robin within same level | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-051 | Lottery: zero total tickets returns -1 | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-052 | Lottery: zero-ticket task never selected across 1000 draws | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-053 | Lottery selection is deterministic for fixed seed | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| UT-SCHED-054 | Lottery: high-ticket task wins more often | SRS-COOP | Unit | PASS | 15-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-001 | FCFS orders 5 tasks correctly by arrival | SRS-COOP | Component | PASS | 17-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-002 | SJF orders 5 tasks correctly by burst | SRS-COOP | Component | PASS | 17-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-003 | Round-Robin fairness: each task gets at least one turn in 6 cycles | SRS-COOP | Component | PASS | 17-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-004 | HRRN aging: high-wait task wins over shorter burst | SRS-COOP | Component | PASS | 17-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-005 | Priority scheduling runs tasks in strict priority order | SRS-COOP | Component | PASS | 17-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-006 | MLQ level isolation: critical→normal→background | SRS-COOP | Component | PASS | 17-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-007 | Lottery determinism: two sessions same seed same sequence | SRS-COOP | Component | PASS | 17-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-008 | FCFS and SJF produce different orderings for same task set | SRS-COOP | Component | PASS | 18-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-009 | SCHED_GetThreadCount matches three CreateTask calls | SRS-COOP | Component | PASS | 18-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-010 | Alive count lifecycle: 3 tasks then all finished | SRS-COOP | Component | PASS | 18-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-011 | Round-Robin yield gives other task a turn | SRS-COOP | Component | PASS | 18-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-012 | Exit: finished task not selected | SRS-COOP | Component | PASS | 18-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-013 | Run count tracks task resumptions | SRS-COOP | Component | PASS | 18-Mar-2025 | Piyush Singh Bhati |
| CT-SCHED-014 | Sum of per-task run counts equals total picks | SRS-COOP | Component | PASS | 18-Mar-2025 | Piyush Singh Bhati |

6.4 Scheduler — Detailed Test Cases

| UT-SCHED-001 | SCHED_Init returns STATUS_OK | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | KMEM_Init(); SCHED_Init() |  |  |
| Expected Result | STATUS_OK | Actual Result | PASS |

| UT-SCHED-002 | After SCHED_Init only idle thread exists (count==1) | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | SCHED_GetThreadCount() after Init |  |  |
| Expected Result | equals 1 | Actual Result | PASS |

| UT-SCHED-003 | Thread count is 1 immediately after Init | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | SCHED_GetThreadCount() |  |  |
| Expected Result | equals 1 | Actual Result | PASS |

| UT-SCHED-004 | PRNG determinism: same seed yields same first draw | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | fx_init twice with PRNG_SEED_DEFAULT; compare first LCG output |  |  |
| Expected Result | r1 equals r2 | Actual Result | PASS |
| Remarks | Fixture-level test |  |  |

| UT-SCHED-005 | ResetAll then Init produces identical state | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | reset_sched twice; compare thread count |  |  |
| Expected Result | Identical | Actual Result | PASS |

| UT-SCHED-006 | THREAD_Create returns STATUS_OK and non-NULL thread | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | THREAD_Create valid params |  |  |
| Expected Result | STATUS_OK && t not equal to NULL | Actual Result | PASS |

| UT-SCHED-007 | THREAD_Create with NULL func returns INVALID_ARGUMENT | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | THREAD_Create(NULL func) |  |  |
| Expected Result | STATUS_ERROR_INVALID_ARGUMENT | Actual Result | PASS |

| UT-SCHED-008 | THREAD_Create at THREAD_MAX_COUNT returns THREAD_LIMIT | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Create 15 threads; create 16th |  |  |
| Expected Result | STATUS_ERROR_THREAD_LIMIT | Actual Result | PASS |

| UT-SCHED-009 | THREAD_Create with stack_size=0 uses THREAD_DEFAULT_STACK | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Create with size=0; check t->stack_size |  |  |
| Expected Result | equals THREAD_DEFAULT_STACK (8192) | Actual Result | PASS |

| UT-SCHED-010 | New thread wake_tick is 0 at creation | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Create thread; check t->wake_tick |  |  |
| Expected Result | equals 0 | Actual Result | PASS |

| UT-SCHED-011 | Thread IDs are strictly increasing | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Create two threads; compare IDs |  |  |
| Expected Result | t2->id > t1->id | Actual Result | PASS |

| UT-SCHED-012 | New thread state is THREAD_STATE_READY | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Create thread; check t->state |  |  |
| Expected Result | THREAD_STATE_READY | Actual Result | PASS |

| UT-SCHED-013 | Thread name stored correctly | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Create with name "mythread"; check t->name |  |  |
| Expected Result | Equals 'mythread' | Actual Result | PASS |

| UT-SCHED-014 | NULL name does not crash and name is non-NULL | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 13-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Create with NULL name; check t->name |  |  |
| Expected Result | Non-NULL string | Actual Result | PASS |

| UT-SCHED-015 | SetTaskPriority updates priority field | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Create; set t->priority=HIGH; check |  |  |
| Expected Result | equals THREAD_PRIORITY_HIGH | Actual Result | PASS |

| UT-SCHED-016 | SetTaskPriority with invalid ID does not crash | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Verify no crash on no-op |  |  |
| Expected Result | No crash | Actual Result | PASS |

| UT-SCHED-017 | SetTaskBurst updates burst_estimate in fixture | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | fx_add; set burst=50; check |  |  |
| Expected Result | equals 50 | Actual Result | PASS |

| UT-SCHED-018 | SetTaskBurst invalid ID does not crash | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | No-op test |  |  |
| Expected Result | No crash | Actual Result | PASS |

| UT-SCHED-019 | SetTaskQueueLevel updates queue_level in fixture | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | fx_add; set queue_level=2; check |  |  |
| Expected Result | equals 2 | Actual Result | PASS |

| UT-SCHED-020 | SetTaskQueueLevel invalid ID does not crash | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | SetTaskQueueLevel with invalid ID; verify no side effect |  |  |
| Expected Result | No crash | Actual Result | PASS |

| UT-SCHED-021 | SetTaskTickets updates tickets field in fixture | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | fx_add; set tickets=20; check |  |  |
| Expected Result | equals 20 | Actual Result | PASS |

| UT-SCHED-022 | SetTaskTickets invalid ID does not crash | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | SetTaskTickets with invalid ID; verify no side effect |  |  |
| Expected Result | No crash | Actual Result | PASS |

| UT-SCHED-023 | THREAD_GetCurrent returns idle thread (id==0) after Init | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | THREAD_GetCurrent(); check ->id |  |  |
| Expected Result | equals 0 | Actual Result | PASS |

| UT-SCHED-024 | Thread count is 1 (idle) immediately before any THREAD_Create | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | After Init; record count; create one thread; verify count still equals 1 (only idle) |  |  |
| Expected Result | equals 1 | Actual Result | PASS |

| UT-SCHED-025 | Thread count increases by 1 per CreateTask | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Record base; Create one; check count |  |  |
| Expected Result | base+1 | Actual Result | PASS |

| UT-SCHED-026 | SCHED_GetUptime does not crash | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Call SCHED_GetUptime() |  |  |
| Expected Result | Returns without crash | Actual Result | PASS |

| UT-SCHED-027 | SCHED_GetThreadCount matches create calls | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 14-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Create 2 threads; check count==base+2 |  |  |
| Expected Result | base+2 | Actual Result | PASS |

| UT-SCHED-028 | PolicyName FCFS returns 'FCFS' | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | fx_policy_name(POL_FCFS) |  |  |
| Expected Result | FCFS | Actual Result | PASS |

| UT-SCHED-029 | PolicyName SJF returns 'SJF' | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | fx_policy_name(POL_SJF) |  |  |
| Expected Result | SJF | Actual Result | PASS |

| UT-SCHED-030 | PolicyName RR returns 'Round-Robin' | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | fx_policy_name(POL_RR) |  |  |
| Expected Result | Round-Robin | Actual Result | PASS |

| UT-SCHED-031 | PolicyName HRRN returns 'HRRN' | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | fx_policy_name(POL_HRRN) |  |  |
| Expected Result | HRRN | Actual Result | PASS |

| UT-SCHED-032 | PolicyName Priority returns 'Priority' | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | fx_policy_name(POL_PRIORITY) |  |  |
| Expected Result | Priority | Actual Result | PASS |

| UT-SCHED-033 | PolicyName MLQ returns 'MLQ' | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | fx_policy_name(POL_MLQ) |  |  |
| Expected Result | MLQ | Actual Result | PASS |

| UT-SCHED-034 | PolicyName Lottery returns 'Lottery' | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | fx_policy_name(POL_LOTTERY) |  |  |
| Expected Result | Lottery | Actual Result | PASS |

| UT-SCHED-035 | PolicyName out-of-range returns 'Unknown' | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | fx_policy_name((fx_policy_t)99) |  |  |
| Expected Result | Unknown | Actual Result | PASS |

| UT-SCHED-036 | FCFS selects task with lowest arrival_order | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Add tasks with arrivals 5/2/8; fx_fcfs() |  |  |
| Expected Result | Selects id=1 (arrival=2) | Actual Result | PASS |

| UT-SCHED-037 | FCFS is consistent for equal arrival times | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Two tasks same arrival; call fx_fcfs() twice |  |  |
| Expected Result | Same result both times | Actual Result | PASS |

| UT-SCHED-038 | SJF selects task with lowest burst estimate | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Tasks burst=100/20/50; fx_sjf() |  |  |
| Expected Result | Selects id=1 (burst=20) | Actual Result | PASS |

| UT-SCHED-039 | SJF is consistent for equal burst times | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Two tasks same burst; call fx_sjf() twice |  |  |
| Expected Result | Same result; result >= 0 | Actual Result | PASS |

| UT-SCHED-040 | Round-Robin cycles through all tasks in order | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Three tasks; call fx_rr() four times |  |  |
| Expected Result | Order: 0,1,2,0 | Actual Result | PASS |

| UT-SCHED-041 | Round-Robin skips FINISHED tasks | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Mark id=1 FINISHED; call fx_rr() twice |  |  |
| Expected Result | Returns 0 then 2 | Actual Result | PASS |

| UT-SCHED-042 | Round-Robin wraps from last to first | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Two tasks; advance to last; call fx_rr() |  |  |
| Expected Result | Wraps to 0 | Actual Result | PASS |

| UT-SCHED-043 | HRRN selects task with higher wait/burst ratio | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Task0 wait=5 burst=10 (ratio 1.5); Task1 wait=20 burst=10 (ratio 3.0) |  |  |
| Expected Result | Selects id=1 | Actual Result | PASS |

| UT-SCHED-044 | HRRN burst=0 treated as 1 (no div-by-zero) | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Task with burst=0; call fx_hrrn() |  |  |
| Expected Result | Returns valid index >= 0 | Actual Result | PASS |

| UT-SCHED-045 | Priority selects numerically lowest priority value | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Tasks prio=3/1/2; fx_priority() |  |  |
| Expected Result | Selects id=1 (prio=1) | Actual Result | PASS |

| UT-SCHED-046 | Priority selection consistent for equal priorities | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Two tasks same priority; call fx_priority() twice |  |  |
| Expected Result | Same result | Actual Result | PASS |

| UT-SCHED-047 | MLQ: level 0 beats levels 1 and 2 | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Tasks in levels 2/0/1; fx_mlq() |  |  |
| Expected Result | Selects level-0 task | Actual Result | PASS |

| UT-SCHED-048 | MLQ: level 1 selected when level 0 absent | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Tasks in levels 2/1; fx_mlq() |  |  |
| Expected Result | Selects level-1 task | Actual Result | PASS |

| UT-SCHED-049 | MLQ: level 2 only when 0 and 1 absent | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Single task in level 2; fx_mlq() |  |  |
| Expected Result | Selects that task | Actual Result | PASS |

| UT-SCHED-050 | MLQ: round-robin within same level | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Two tasks level=1; call fx_mlq() three times |  |  |
| Expected Result | 0, 1, 0 | Actual Result | PASS |

| UT-SCHED-051 | Lottery: zero total tickets returns -1 | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Two tasks with 0 tickets; fx_lottery() |  |  |
| Expected Result | Returns -1 | Actual Result | PASS |

| UT-SCHED-052 | Lottery: zero-ticket task never selected across 1000 draws | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Task0 0 tickets; Task1 10 tickets; 1000 draws |  |  |
| Expected Result | Task0 never selected | Actual Result | PASS |

| UT-SCHED-053 | Lottery selection is deterministic for fixed seed | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Two identical fx_sched_t; compare first 5 picks |  |  |
| Expected Result | Sequences identical | Actual Result | PASS |

| UT-SCHED-054 | Lottery: high-ticket task wins more often | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 15-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Task0 1 ticket; Task1 99 tickets; 1000 draws |  |  |
| Expected Result | wins[1] > wins[0] | Actual Result | PASS |

| CT-SCHED-001 | FCFS orders 5 tasks correctly by arrival | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 17-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | 5 tasks shuffled arrivals; select all via FCFS |  |  |
| Expected Result | Order: 1,2,3,4,5 | Actual Result | PASS |

| CT-SCHED-002 | SJF orders 5 tasks correctly by burst | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 17-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | 5 tasks varying burst; select all via SJF |  |  |
| Expected Result | Order by burst ascending | Actual Result | PASS |

| CT-SCHED-003 | Round-Robin fairness: each task gets at least one turn in 6 cycles | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 17-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | 3 tasks; 6 fx_rr() calls; count per task |  |  |
| Expected Result | Each seen >= 1 time | Actual Result | PASS |

| CT-SCHED-004 | HRRN aging: high-wait task wins over shorter burst | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 17-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Task0 burst=5 wait=50; Task1 burst=1 wait=1 |  |  |
| Expected Result | Selects Task0 (ratio 11 > 2) | Actual Result | PASS |

| CT-SCHED-005 | Priority scheduling runs tasks in strict priority order | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 17-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | 5 tasks priorities 5..1; select all |  |  |
| Expected Result | Runs in order 1,2,3,4,5 | Actual Result | PASS |

| CT-SCHED-006 | MLQ level isolation: critical→normal→background | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 17-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Tasks levels 2/1/0; select three times |  |  |
| Expected Result | Level 0 first, then 1, then 2 | Actual Result | PASS |

| CT-SCHED-007 | Lottery determinism: two sessions same seed same sequence | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 17-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Two identical schedulers; compare 10 picks |  |  |
| Expected Result | All picks identical | Actual Result | PASS |

| CT-SCHED-008 | FCFS and SJF produce different orderings for same task set | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 18-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Tasks: burst=100 arrives 1st; burst=10 arrives 3rd |  |  |
| Expected Result | FCFS and SJF pick differently | Actual Result | PASS |

| CT-SCHED-009 | SCHED_GetThreadCount matches three CreateTask calls | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 18-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Record base; Create 3 threads; check count |  |  |
| Expected Result | base+3 | Actual Result | PASS |

| CT-SCHED-010 | Alive count lifecycle: 3 tasks then all finished | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 18-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Add 3 fx tasks; count ready (3); mark finished; count (0) |  |  |
| Expected Result | Counts correct | Actual Result | PASS |

| CT-SCHED-011 | Round-Robin yield gives other task a turn | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 18-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Two tasks; first fx_rr returns A; second returns B |  |  |
| Expected Result | A not equal to B | Actual Result | PASS |

| CT-SCHED-012 | Exit: finished task not selected | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 18-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | One task; mark FINISHED; fx_rr() |  |  |
| Expected Result | Returns -1 | Actual Result | PASS |

| CT-SCHED-013 | Run count tracks task resumptions | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 18-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | Two tasks; 3 RR ticks; count task0 selections |  |  |
| Expected Result | run_count >= 1 | Actual Result | PASS |

| CT-SCHED-014 | Sum of per-task run counts equals total picks | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Piyush Singh Bhati | Date: 18-Mar-2025 | Scheduler |
| Prerequisites | Host build; KMEM_Init and SCHED_Init called; thread.c and kmem.c linked; stubs active |  |  |
| Test Procedure | 3 tasks; 12 RR picks; sum per_task counts |  |  |
| Expected Result | Sum equals 12 | Actual Result | PASS |

7. Context Switch, Exceptions, KAPI & System Tests

| Tester: Aashma | Role: System Components Developer | Test Cases: 48 | Status: ALL PASS |
| --- | --- | --- | --- |

7.1 Context Switch — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| UT-CTX-001 | ctx_a.sp is non-zero after round-trip context switch | SRS-COOP | Unit | PASS | 13-Mar-2025 | Aashma |
| UT-CTX-002 | Frame pointer restored correctly (no fault after switch) | SRS-COOP | Unit | PASS | 13-Mar-2025 | Aashma |
| UT-CTX-003 | Link register restores to correct return address | SRS-COOP | Unit | PASS | 13-Mar-2025 | Aashma |
| UT-CTX-004 | cpu_context_switch saves non-zero SP into old_ctx | SRS-COOP | Unit | PASS | 13-Mar-2025 | Aashma |
| UT-CTX-005 | SP correctly loaded from new_ctx before register restore | SRS-COOP | Unit | PASS | 13-Mar-2025 | Aashma |
| UT-CTX-006 | Callee-saved x19-x28 saved before SP restore (assembly order) | SRS-COOP | Unit | PASS | 13-Mar-2025 | Aashma |
| UT-CTX-007 | Full round-trip A→B→A resumes at correct instruction | SRS-COOP | Unit | PASS | 13-Mar-2025 | Aashma |
| UT-CTX-008 | Caller-saved x0-x18 not preserved across switch (by design) | SRS-COOP | Unit | PASS | 13-Mar-2025 | Aashma |

7.2 Context Switch — Detailed Test Cases

| UT-CTX-001 | ctx_a.sp is non-zero after round-trip context switch | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 13-Mar-2025 | Context Switch |
| Prerequisites | QEMU virt running; cpu_context_switch in context.S; _thread_entry_trampoline available |  |  |
| Test Procedure | Run UT-CTX-007 first; check ctx_a.sp |  |  |
| Expected Result | ctx_a.sp not equal to 0 | Actual Result | PASS |
| Remarks | Must run after CTX-007 |  |  |

| UT-CTX-002 | Frame pointer restored correctly (no fault after switch) | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 13-Mar-2025 | Context Switch |
| Prerequisites | QEMU virt running; cpu_context_switch in context.S; _thread_entry_trampoline available |  |  |
| Test Procedure | Verify reaching this point after CTX-007 ran |  |  |
| Expected Result | No fault | Actual Result | PASS |
| Remarks | Structural |  |  |

| UT-CTX-003 | Link register restores to correct return address | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 13-Mar-2025 | Context Switch |
| Prerequisites | QEMU virt running; cpu_context_switch in context.S; _thread_entry_trampoline available |  |  |
| Test Procedure | After switch back from ctx_b_func, resume at correct PC |  |  |
| Expected Result | No fault; correct resume | Actual Result | PASS |
| Remarks | Structural |  |  |

| UT-CTX-004 | cpu_context_switch saves non-zero SP into old_ctx | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 13-Mar-2025 | Context Switch |
| Prerequisites | QEMU virt running; cpu_context_switch in context.S; _thread_entry_trampoline available |  |  |
| Test Procedure | Switch to ctx4_body; it switches back; verify ctx4_save.sp!=0 and ctx4_returned==1 |  |  |
| Expected Result | Both conditions true | Actual Result | PASS |
| Remarks | Real round-trip |  |  |

| UT-CTX-005 | SP correctly loaded from new_ctx before register restore | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 13-Mar-2025 | Context Switch |
| Prerequisites | QEMU virt running; cpu_context_switch in context.S; _thread_entry_trampoline available |  |  |
| Test Procedure | Context B ran from ctx_stack_b without corrupting caller stack |  |  |
| Expected Result | No fault | Actual Result | PASS |
| Remarks | Structural |  |  |

| UT-CTX-006 | Callee-saved x19-x28 saved before SP restore (assembly order) | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 13-Mar-2025 | Context Switch |
| Prerequisites | QEMU virt running; cpu_context_switch in context.S; _thread_entry_trampoline available |  |  |
| Test Procedure | Code inspection of context.S: stp before ldr sp |  |  |
| Expected Result | Verified by inspection | Actual Result | PASS |
| Remarks | Assembly structural |  |  |

| UT-CTX-007 | Full round-trip A→B→A resumes at correct instruction | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 13-Mar-2025 | Context Switch |
| Prerequisites | QEMU virt running; cpu_context_switch in context.S; _thread_entry_trampoline available |  |  |
| Test Procedure | Set step=1; switch to ctx_b_func; it sets step=2 and switches back; verify step==2 |  |  |
| Expected Result | ctx_test_step equals 2 | Actual Result | PASS |
| Remarks | Real context switch |  |  |

| UT-CTX-008 | Caller-saved x0-x18 not preserved across switch (by design) | PASS | Unit |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 13-Mar-2025 | Context Switch |
| Prerequisites | QEMU virt running; cpu_context_switch in context.S; _thread_entry_trampoline available |  |  |
| Test Procedure | Verify by assembly inspection of context.S |  |  |
| Expected Result | Expected: only callee-saved regs saved | Actual Result | PASS |
| Remarks | Structural |  |  |

7.3 Exception Handling — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| CT-EXC-001 | VBAR_EL1 points to _vector_table | SRS-COOP | Component | PASS | 14-Mar-2025 | Aashma |
| CT-EXC-002 | ESR_EL1 ELR_EL1 FAR_EL1 readable without fault | SRS-COOP | Component | PASS | 14-Mar-2025 | Aashma |
| CT-EXC-003 | HAL_Exception_Handler called with correct args (structural) | SRS-COOP | Component | PASS | 14-Mar-2025 | Aashma |
| CT-EXC-004 | Processor enters WFE after exception handler returns | SRS-COOP | Component | PASS | 14-Mar-2025 | Aashma |

7.4 Exception Handling — Detailed Test Cases

| CT-EXC-001 | VBAR_EL1 points to _vector_table | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 14-Mar-2025 | Exception Handling |
| Prerequisites | QEMU virt running; vectors.S loaded; VBAR_EL1 set to _vector_table |  |  |
| Test Procedure | Read VBAR_EL1; compare to &_vector_table |  |  |
| Expected Result | vbar equals (uint64_t)&_vector_table | Actual Result | PASS |

| CT-EXC-002 | ESR_EL1 ELR_EL1 FAR_EL1 readable without fault | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 14-Mar-2025 | Exception Handling |
| Prerequisites | QEMU virt running; vectors.S loaded; VBAR_EL1 set to _vector_table |  |  |
| Test Procedure | mrs esr_el1; mrs elr_el1; mrs far_el1 |  |  |
| Expected Result | All reads succeed; no fault | Actual Result | PASS |

| CT-EXC-003 | HAL_Exception_Handler called with correct args (structural) | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 14-Mar-2025 | Exception Handling |
| Prerequisites | QEMU virt running; vectors.S loaded; VBAR_EL1 set to _vector_table |  |  |
| Test Procedure | Verify exception_names[] in main.c and vectors.S branch |  |  |
| Expected Result | Structural verification | Actual Result | PASS |
| Remarks | Code inspection |  |  |

| CT-EXC-004 | Processor enters WFE after exception handler returns | PASS | Component |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 14-Mar-2025 | Exception Handling |
| Prerequisites | QEMU virt running; vectors.S loaded; VBAR_EL1 set to _vector_table |  |  |
| Test Procedure | Code inspection of _exception_handler in vectors.S |  |  |
| Expected Result | WFE loop present | Actual Result | PASS |
| Remarks | Structural; cannot test without real exception |  |  |

7.5 Kernel API — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| CT-KAPI-001 | Nested IRQ save/restore cycle completes without crash | FR-004 | Component | PASS | 14-Mar-2025 | Aashma |
| CT-KAPI-002 | Timer measurement: t1 >= t0 after DelayUs | FR-005 | Component | PASS | 14-Mar-2025 | Aashma |
| CT-KAPI-003 | HAL_UART_PutString does not crash (KAPI_Log simulation) | FR-022 | Component | PASS | 14-Mar-2025 | Aashma |
| CT-KAPI-004 | Cache flush (no-op on host) does not corrupt memory pattern | FR-003 | Component | PASS | 14-Mar-2025 | Aashma |

7.6 Kernel API — Detailed Test Cases

| CT-KAPI-001 | Nested IRQ save/restore cycle completes without crash | PASS | Component |
| --- | --- | --- | --- |
| Req: FR-004 | Tester: Aashma | Date: 14-Mar-2025 | Kernel API |
| Prerequisites | Host build; arch_stub/uart_stub/timer_stub linked; Unity initialized |  |  |
| Test Procedure | arch_irq_save (outer); arch_irq_save (inner); restore inner; restore outer |  |  |
| Expected Result | No crash; returns | Actual Result | PASS |
| Remarks | Stubs are no-ops on host |  |  |

| CT-KAPI-002 | Timer measurement: t1 >= t0 after DelayUs | PASS | Component |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Aashma | Date: 14-Mar-2025 | Kernel API |
| Prerequisites | Host build; arch_stub/uart_stub/timer_stub linked; Unity initialized |  |  |
| Test Procedure | t0=GetTicks(); DelayUs(1000); t1=GetTicks(); compare |  |  |
| Expected Result | t1 >= t0 | Actual Result | PASS |

| CT-KAPI-003 | HAL_UART_PutString does not crash (KAPI_Log simulation) | PASS | Component |
| --- | --- | --- | --- |
| Req: FR-022 | Tester: Aashma | Date: 14-Mar-2025 | Kernel API |
| Prerequisites | Host build; arch_stub/uart_stub/timer_stub linked; Unity initialized |  |  |
| Test Procedure | HAL_UART_PutString("[CT-KAPI-003] message\n") |  |  |
| Expected Result | No crash | Actual Result | PASS |

| CT-KAPI-004 | Cache flush (no-op on host) does not corrupt memory pattern | PASS | Component |
| --- | --- | --- | --- |
| Req: FR-003 | Tester: Aashma | Date: 14-Mar-2025 | Kernel API |
| Prerequisites | Host build; arch_stub/uart_stub/timer_stub linked; Unity initialized |  |  |
| Test Procedure | Write pattern; compiler barrier; verify pattern |  |  |
| Expected Result | Pattern preserved | Actual Result | PASS |

7.7 System Boot — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| ST-BOOT-001 | Kernel boots and reaches test runner | FR-001 | System | PASS | 15-Mar-2025 | Aashma |
| ST-BOOT-002 | Execution level at kernel entry is EL1 | FR-001 | System | PASS | 15-Mar-2025 | Aashma |
| ST-BOOT-003 | BSS section is zeroed by boot code | FR-001 | System | PASS | 15-Mar-2025 | Aashma |
| ST-BOOT-004 | Stack functional: deep recursive local arrays work | FR-001 | System | PASS | 15-Mar-2025 | Aashma |
| ST-BOOT-005 | UART operational before test suite runs | FR-021 | System | PASS | 15-Mar-2025 | Aashma |
| ST-BOOT-006 | Exception vectors installed before hardware init | FR-004 | System | PASS | 15-Mar-2025 | Aashma |

7.8 System Boot — Detailed Test Cases

| ST-BOOT-001 | Kernel boots and reaches test runner | PASS | System |
| --- | --- | --- | --- |
| Req: FR-001 | Tester: Aashma | Date: 15-Mar-2025 | System Boot |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Observe test runner execution on QEMU serial output |  |  |
| Expected Result | Test runner prints banner | Actual Result | PASS |

| ST-BOOT-002 | Execution level at kernel entry is EL1 | PASS | System |
| --- | --- | --- | --- |
| Req: FR-001 | Tester: Aashma | Date: 15-Mar-2025 | System Boot |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Read CurrentEL; extract bits [3:2] |  |  |
| Expected Result | level equals 1 | Actual Result | PASS |

| ST-BOOT-003 | BSS section is zeroed by boot code | PASS | System |
| --- | --- | --- | --- |
| Req: FR-001 | Tester: Aashma | Date: 15-Mar-2025 | System Boot |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Check global uninitialized variable bss_test_var |  |  |
| Expected Result | equals 0 | Actual Result | PASS |

| ST-BOOT-004 | Stack functional: deep recursive local arrays work | PASS | System |
| --- | --- | --- | --- |
| Req: FR-001 | Tester: Aashma | Date: 15-Mar-2025 | System Boot |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Call deep_stack_func(4); verify returns non-negative |  |  |
| Expected Result | No fault; result >= 0 | Actual Result | PASS |

| ST-BOOT-005 | UART operational before test suite runs | PASS | System |
| --- | --- | --- | --- |
| Req: FR-021 | Tester: Aashma | Date: 15-Mar-2025 | System Boot |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | HAL_UART_PutString output visible |  |  |
| Expected Result | String appears on serial | Actual Result | PASS |

| ST-BOOT-006 | Exception vectors installed before hardware init | PASS | System |
| --- | --- | --- | --- |
| Req: FR-004 | Tester: Aashma | Date: 15-Mar-2025 | System Boot |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Read VBAR_EL1 and compare to &_vector_table |  |  |
| Expected Result | Match | Actual Result | PASS |

7.9 System Init — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| ST-INIT-001 | All subsystems initialize successfully | FR-001,FR-002,FR-005,FR-016 | System | PASS | 15-Mar-2025 | Aashma |
| ST-INIT-002 | Timer frequency in range 50–100 MHz after init | FR-005 | System | PASS | 15-Mar-2025 | Aashma |
| ST-INIT-003 | Heap size reported matches linker symbols | FR-016,FR-020 | System | PASS | 15-Mar-2025 | Aashma |
| ST-INIT-004 | Normal RAM read/write after MMU init | FR-002 | System | PASS | 15-Mar-2025 | Aashma |
| ST-INIT-005 | Timer readable after MMU init | FR-005 | System | PASS | 15-Mar-2025 | Aashma |

7.10 System Init — Detailed Test Cases

| ST-INIT-001 | All subsystems initialize successfully | PASS | System |
| --- | --- | --- | --- |
| Req: FR-001,FR-002,FR-005,FR-016 | Tester: Aashma | Date: 15-Mar-2025 | System Init |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Kernel reaches test runner (all inits passed) |  |  |
| Expected Result | No fault before runner | Actual Result | PASS |

| ST-INIT-002 | Timer frequency in range 50–100 MHz after init | PASS | System |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Aashma | Date: 15-Mar-2025 | System Init |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | HAL_Timer_GetFreq() |  |  |
| Expected Result | 50000000 <= freq <= 100000000 | Actual Result | PASS |

| ST-INIT-003 | Heap size reported matches linker symbols | PASS | System |
| --- | --- | --- | --- |
| Req: FR-016,FR-020 | Tester: Aashma | Date: 15-Mar-2025 | System Init |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | KMEM_GetStats; compare to _heap_end-_heap_start |  |  |
| Expected Result | 0 < heap_total <= linker range | Actual Result | PASS |

| ST-INIT-004 | Normal RAM read/write after MMU init | PASS | System |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Aashma | Date: 15-Mar-2025 | System Init |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Write 0x12345678 to 0x40110000; read back |  |  |
| Expected Result | Match | Actual Result | PASS |

| ST-INIT-005 | Timer readable after MMU init | PASS | System |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Aashma | Date: 15-Mar-2025 | System Init |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | HAL_Timer_GetTicks() |  |  |
| Expected Result | ticks > 0 | Actual Result | PASS |

7.11 System API — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| ST-API-001 | UART API tests all passed | FR-021,FR-022 | System | PASS | 17-Mar-2025 | Aashma |
| ST-API-002 | Memory API tests all passed | FR-016,FR-020 | System | PASS | 17-Mar-2025 | Aashma |
| ST-API-003 | KAPI tests all passed | FR-004,FR-005 | System | PASS | 17-Mar-2025 | Aashma |
| ST-API-004 | Scheduler API: SCHED_Init and SCHED_GetThreadCount functional | SRS-COOP | System | PASS | 17-Mar-2025 | Aashma |
| ST-API-005 | All prior API test modules pass with 0 failures | SRS-COOP | System | PASS | 17-Mar-2025 | Aashma |

7.12 System API — Detailed Test Cases

| ST-API-001 | UART API tests all passed | PASS | System |
| --- | --- | --- | --- |
| Req: FR-021,FR-022 | Tester: Aashma | Date: 17-Mar-2025 | System API |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | All UT-UART tests ran without failure |  |  |
| Expected Result | PASS aggregate | Actual Result | PASS |
| Remarks | Structural summary |  |  |

| ST-API-002 | Memory API tests all passed | PASS | System |
| --- | --- | --- | --- |
| Req: FR-016,FR-020 | Tester: Aashma | Date: 17-Mar-2025 | System API |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | All UT-MEM and CT-MEM tests ran |  |  |
| Expected Result | PASS aggregate | Actual Result | PASS |
| Remarks | Structural summary |  |  |

| ST-API-003 | KAPI tests all passed | PASS | System |
| --- | --- | --- | --- |
| Req: FR-004,FR-005 | Tester: Aashma | Date: 17-Mar-2025 | System API |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | All CT-KAPI tests ran |  |  |
| Expected Result | PASS aggregate | Actual Result | PASS |
| Remarks | Structural summary |  |  |

| ST-API-004 | Scheduler API: SCHED_Init and SCHED_GetThreadCount functional | PASS | System |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 17-Mar-2025 | System API |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | SCHED_Init(); SCHED_GetThreadCount() >= 1 |  |  |
| Expected Result | Count >= 1 | Actual Result | PASS |

| ST-API-005 | All prior API test modules pass with 0 failures | PASS | System |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 17-Mar-2025 | System API |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Review total fail count from prior modules |  |  |
| Expected Result | fail count equals 0 | Actual Result | PASS |
| Remarks | Structural |  |  |

7.13 System Benchmark — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| ST-BENCH-001 | Timer measurement baseline established | FR-012 | System | PASS | 17-Mar-2025 | Aashma |
| ST-BENCH-002 | Heap available for benchmark allocations | FR-016 | System | PASS | 17-Mar-2025 | Aashma |
| ST-BENCH-003 | KMEM_GetStats NULL guard does not crash | FR-020 | System | PASS | 17-Mar-2025 | Aashma |
| ST-BENCH-004 | Bump allocator handles sequential allocations | FR-016 | System | PASS | 17-Mar-2025 | Aashma |
| ST-BENCH-005 | Timer resolution sufficient for benchmarking | FR-005 | System | PASS | 17-Mar-2025 | Aashma |
| ST-BENCH-006 | Scheduling overhead within acceptable bounds | FR-012 | System | PASS | 18-Mar-2025 | Aashma |
| ST-BENCH-007 | Cache coherency maintained during benchmark | FR-002 | System | PASS | 18-Mar-2025 | Aashma |
| ST-BENCH-008 | Arena allocator available for tensor benchmarks | FR-016 | System | PASS | 18-Mar-2025 | Aashma |
| ST-BENCH-009 | No OOM condition: heap has free space after bench allocs | FR-020 | System | PASS | 18-Mar-2025 | Aashma |
| ST-BENCH-010 | Benchmark allocations complete without panic | FR-016 | System | PASS | 18-Mar-2025 | Aashma |

7.14 System Benchmark — Detailed Test Cases

| ST-BENCH-001 | Timer measurement baseline established | PASS | System |
| --- | --- | --- | --- |
| Req: FR-012 | Tester: Aashma | Date: 17-Mar-2025 | System Benchmark |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Timer operational for benchmarking |  |  |
| Expected Result | Timer functional | Actual Result | PASS |
| Remarks | Structural |  |  |

| ST-BENCH-002 | Heap available for benchmark allocations | PASS | System |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Aashma | Date: 17-Mar-2025 | System Benchmark |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Heap initialized and usable |  |  |
| Expected Result | Heap functional | Actual Result | PASS |
| Remarks | Structural |  |  |

| ST-BENCH-003 | KMEM_GetStats NULL guard does not crash | PASS | System |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Aashma | Date: 17-Mar-2025 | System Benchmark |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | KMEM_GetStats(NULL) then KMEM_GetStats(&st); check heap_total > 0 |  |  |
| Expected Result | No crash; heap_total > 0 | Actual Result | PASS |

| ST-BENCH-004 | Bump allocator handles sequential allocations | PASS | System |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Aashma | Date: 17-Mar-2025 | System Benchmark |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Multiple sequential allocs succeed |  |  |
| Expected Result | All succeed | Actual Result | PASS |
| Remarks | Structural |  |  |

| ST-BENCH-005 | Timer resolution sufficient for benchmarking | PASS | System |
| --- | --- | --- | --- |
| Req: FR-005 | Tester: Aashma | Date: 17-Mar-2025 | System Benchmark |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Timer frequency > 50 MHz verified |  |  |
| Expected Result | Adequate resolution | Actual Result | PASS |
| Remarks | Structural |  |  |

| ST-BENCH-006 | Scheduling overhead within acceptable bounds | PASS | System |
| --- | --- | --- | --- |
| Req: FR-012 | Tester: Aashma | Date: 18-Mar-2025 | System Benchmark |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Scheduler initialized; cooperative yield available |  |  |
| Expected Result | No fault | Actual Result | PASS |
| Remarks | Structural |  |  |

| ST-BENCH-007 | Cache coherency maintained during benchmark | PASS | System |
| --- | --- | --- | --- |
| Req: FR-002 | Tester: Aashma | Date: 18-Mar-2025 | System Benchmark |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | MMU and caches enabled; normal WB memory in use |  |  |
| Expected Result | Caches enabled | Actual Result | PASS |
| Remarks | Structural |  |  |

| ST-BENCH-008 | Arena allocator available for tensor benchmarks | PASS | System |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Aashma | Date: 18-Mar-2025 | System Benchmark |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | KMEM_ArenaCreate accessible |  |  |
| Expected Result | Arena functional | Actual Result | PASS |
| Remarks | Structural |  |  |

| ST-BENCH-009 | No OOM condition: heap has free space after bench allocs | PASS | System |
| --- | --- | --- | --- |
| Req: FR-020 | Tester: Aashma | Date: 18-Mar-2025 | System Benchmark |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | KMEM_GetFreeSpace() > 0 |  |  |
| Expected Result | Free space remains | Actual Result | PASS |

| ST-BENCH-010 | Benchmark allocations complete without panic | PASS | System |
| --- | --- | --- | --- |
| Req: FR-016 | Tester: Aashma | Date: 18-Mar-2025 | System Benchmark |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | All bench allocs return; no fault |  |  |
| Expected Result | No fault | Actual Result | PASS |
| Remarks | Structural |  |  |

7.15 System Memory Stress — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| ST-MEM-STRESS-001 | heap_peak never exceeds heap_total | FR-019 | System | PASS | 18-Mar-2025 | Aashma |
| ST-MEM-STRESS-002 | ArenaReset brings used back to 0 | FR-017 | System | PASS | 18-Mar-2025 | Aashma |
| ST-MEM-STRESS-003 | heap_peak strictly less than heap_total (no full exhaustion) | FR-019 | System | PASS | 18-Mar-2025 | Aashma |

7.16 System Memory Stress — Detailed Test Cases

| ST-MEM-STRESS-001 | heap_peak never exceeds heap_total | PASS | System |
| --- | --- | --- | --- |
| Req: FR-019 | Tester: Aashma | Date: 18-Mar-2025 | System Memory Stress |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | KMEM_GetStats; verify heap_peak <= heap_total |  |  |
| Expected Result | Invariant holds | Actual Result | PASS |

| ST-MEM-STRESS-002 | ArenaReset brings used back to 0 | PASS | System |
| --- | --- | --- | --- |
| Req: FR-017 | Tester: Aashma | Date: 18-Mar-2025 | System Memory Stress |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | ArenaCreate(4096); ArenaAlloc(1024,8); ArenaReset; check used==0 |  |  |
| Expected Result | used equals 0 | Actual Result | PASS |

| ST-MEM-STRESS-003 | heap_peak strictly less than heap_total (no full exhaustion) | PASS | System |
| --- | --- | --- | --- |
| Req: FR-019 | Tester: Aashma | Date: 18-Mar-2025 | System Memory Stress |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | KMEM_GetStats; verify heap_peak < heap_total |  |  |
| Expected Result | heap_peak < heap_total | Actual Result | PASS |
| Remarks | Fragile if heap is exactly full |  |  |

7.17 System Stability — Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| ST-STAB-001 | System stable after all QEMU test modules execute | SRS-COOP | System | PASS | 18-Mar-2025 | Aashma |
| ST-STAB-002 | No unexpected exceptions during test run | SRS-COOP | System | PASS | 18-Mar-2025 | Aashma |
| ST-STAB-003 | QEMU exits cleanly via semihosting on test completion | SRS-COOP | System | PASS | 18-Mar-2025 | Aashma |

7.18 System Stability — Detailed Test Cases

| ST-STAB-001 | System stable after all QEMU test modules execute | PASS | System |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 18-Mar-2025 | System Stability |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Verify test runner reaches summary line without fault |  |  |
| Expected Result | Reaches [SUITE] line | Actual Result | PASS |
| Remarks | Structural |  |  |

| ST-STAB-002 | No unexpected exceptions during test run | PASS | System |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 18-Mar-2025 | System Stability |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Monitor serial for !!! EXCEPTION output |  |  |
| Expected Result | No exception output | Actual Result | PASS |
| Remarks | Structural |  |  |

| ST-STAB-003 | QEMU exits cleanly via semihosting on test completion | PASS | System |
| --- | --- | --- | --- |
| Req: SRS-COOP | Tester: Aashma | Date: 18-Mar-2025 | System Stability |
| Prerequisites | QEMU virt running; full kernel boot sequence complete; all HAL subsystems initialized |  |  |
| Test Procedure | Verify HLT #0xF000 semihosting exit fires |  |  |
| Expected Result | QEMU exits with code 0 | Actual Result | PASS |

Appendix A — Requirement Traceability Matrix

| Requirement ID | Linked Test Cases | Count | Covered |
| --- | --- | --- | --- |
| FR-001 | ST-BOOT-001, ST-BOOT-002, ST-BOOT-003, ST-BOOT-004, ST-INIT-001 | 5 | YES |
| FR-002 | UT-MMU-001, UT-MMU-002, UT-MMU-003, UT-MMU-004, UT-MMU-005, UT-MMU-006, UT-MMU-007, UT-MMU-008, UT-MMU-009, UT-MMU-010, UT-MMU-011, UT-MMU-012, UT-MMU-014, UT-MMU-016, UT-MMU-017, ST-INIT-001, ST-INIT-004, ST-BENCH-007 | 18 | YES |
| FR-003 | UT-MMU-013, UT-MMU-015, CT-KAPI-004 | 3 | YES |
| FR-004 | CT-KAPI-001, ST-BOOT-006, ST-API-003 | 3 | YES |
| FR-005 | UT-TIMER-001, UT-TIMER-002, UT-TIMER-003, UT-TIMER-004, UT-TIMER-005, UT-TIMER-006, UT-TIMER-009, UT-TIMER-010, UT-TIMER-012, UT-TIMER-013, UT-TIMER-014, UT-TIMER-015, UT-TIMER-016, UT-TIMER-017, CT-KAPI-002, ST-INIT-001, ST-INIT-002, ST-INIT-005, ST-API-003, ST-BENCH-005 | 20 | YES |
| FR-012 | UT-TIMER-007, UT-TIMER-008, UT-TIMER-011, ST-BENCH-001, ST-BENCH-006 | 5 | YES |
| FR-016 | UT-MEM-001, UT-MEM-002, UT-MEM-003, UT-MEM-005, UT-MEM-006, UT-MEM-007, UT-MEM-011, UT-MEM-012, UT-MEM-013, UT-MEM-014, CT-MEM-001, ST-INIT-001, ST-INIT-003, ST-API-002, ST-BENCH-002, ST-BENCH-004, ST-BENCH-008, ST-BENCH-010 | 18 | YES |
| FR-017 | UT-MEM-016, UT-MEM-017, UT-MEM-026, UT-MEM-027, UT-MEM-029, CT-MEM-001, CT-MEM-003, CT-MEM-007, ST-MEM-STRESS-002 | 9 | YES |
| FR-018 | UT-MEM-008, UT-MEM-009, UT-MEM-010, UT-MEM-015, UT-MEM-035, UT-MEM-036, UT-MEM-037, UT-MEM-038, UT-MEM-039, UT-MEM-040, UT-MEM-041, UT-MEM-042, UT-MEM-043, UT-MEM-044, UT-MEM-045, CT-MEM-002, CT-MEM-005, CT-MEM-006, CT-MEM-007 | 19 | YES |
| FR-019 | UT-MEM-024, UT-MEM-025, UT-MEM-028, CT-MEM-004, ST-MEM-STRESS-001, ST-MEM-STRESS-003 | 6 | YES |
| FR-020 | UT-MEM-004, UT-MEM-018, UT-MEM-019, UT-MEM-020, UT-MEM-021, UT-MEM-022, UT-MEM-023, UT-MEM-030, UT-MEM-031, UT-MEM-032, UT-MEM-033, UT-MEM-034, CT-MEM-001, ST-INIT-003, ST-API-002, ST-BENCH-003, ST-BENCH-009 | 17 | YES |
| FR-021 | UT-UART-001, UT-UART-002, UT-UART-003, UT-UART-004, UT-UART-005, UT-UART-006, UT-UART-007, UT-UART-008, ST-BOOT-005, ST-API-001 | 10 | YES |
| FR-022 | UT-UART-009, UT-UART-010, UT-UART-011, UT-UART-012, UT-UART-013, UT-UART-014, UT-UART-015, UT-UART-016, UT-UART-017, UT-UART-018, UT-UART-019, UT-UART-020, UT-UART-021, CT-KAPI-003, ST-API-001 | 15 | YES |
| NFR-001 | UT-TYPES-001, UT-TYPES-002, UT-TYPES-003, UT-TYPES-004, UT-TYPES-005, UT-TYPES-006, UT-TYPES-007, UT-TYPES-008, UT-TYPES-009, UT-TYPES-010, UT-TYPES-011, UT-TYPES-012, UT-TYPES-013, UT-TYPES-014, UT-TYPES-015, UT-TYPES-016, UT-TYPES-017 | 17 | YES |
| NFR-002 | UT-STAT-001, UT-STAT-002, UT-STAT-003, UT-STAT-004, UT-STAT-005, UT-STAT-006 | 6 | YES |
| SRS-COOP | UT-SCHED-001, UT-SCHED-002, UT-SCHED-003, UT-SCHED-004, UT-SCHED-005, UT-SCHED-006, UT-SCHED-007, UT-SCHED-008, UT-SCHED-009, UT-SCHED-010, UT-SCHED-011, UT-SCHED-012, UT-SCHED-013, UT-SCHED-014, UT-SCHED-015, UT-SCHED-016, UT-SCHED-017, UT-SCHED-018, UT-SCHED-019, UT-SCHED-020, UT-SCHED-021, UT-SCHED-022, UT-SCHED-023, UT-SCHED-024, UT-SCHED-025, UT-SCHED-026, UT-SCHED-027, UT-SCHED-028, UT-SCHED-029, UT-SCHED-030, UT-SCHED-031, UT-SCHED-032, UT-SCHED-033, UT-SCHED-034, UT-SCHED-035, UT-SCHED-036, UT-SCHED-037, UT-SCHED-038, UT-SCHED-039, UT-SCHED-040, UT-SCHED-041, UT-SCHED-042, UT-SCHED-043, UT-SCHED-044, UT-SCHED-045, UT-SCHED-046, UT-SCHED-047, UT-SCHED-048, UT-SCHED-049, UT-SCHED-050, UT-SCHED-051, UT-SCHED-052, UT-SCHED-053, UT-SCHED-054, CT-SCHED-001, CT-SCHED-002, CT-SCHED-003, CT-SCHED-004, CT-SCHED-005, CT-SCHED-006, CT-SCHED-007, CT-SCHED-008, CT-SCHED-009, CT-SCHED-010, CT-SCHED-011, CT-SCHED-012, CT-SCHED-013, CT-SCHED-014, UT-CTX-001, UT-CTX-002, UT-CTX-003, UT-CTX-004, UT-CTX-005, UT-CTX-006, UT-CTX-007, UT-CTX-008, CT-EXC-001, CT-EXC-002, CT-EXC-003, CT-EXC-004, ST-API-004, ST-API-005, ST-STAB-001, ST-STAB-002, ST-STAB-003 | 85 | YES |

Appendix B — Glossary of Terms

| Term | Definition |
| --- | --- |
| AArch64 | 64-bit execution state of the ARMv8-A architecture |
| BSP | Board Support Package — hardware initialisation code |
| CT | Component Test — verifies interaction between two or more modules |
| DCache | Data Cache — L1/L2 cache for data accesses |
| DSB | Data Synchronisation Barrier — ARM memory barrier instruction |
| EL1 | Exception Level 1 — the kernel privilege level in AArch64 |
| FIFO | First-In First-Out — queue data structure used in UART |
| GIC | Generic Interrupt Controller (GICv3) |
| HAL | Hardware Abstraction Layer — platform-independent driver interface |
| HRRN | Highest Response Ratio Next — ageing-based scheduling policy |
| ISB | Instruction Synchronisation Barrier |
| KAPI | Kernel API — high-level interface to kernel services |
| LCG | Linear Congruential Generator — PRNG used in Lottery scheduler |
| MAIR | Memory Attribute Indirection Register |
| MLQ | Multi-Level Queue — hierarchical scheduling policy |
| MMU | Memory Management Unit |
| PASS | Test result indicating all assertions succeeded |
| PTE | Page Table Entry |
| QEMU | Quick Emulator — open-source hardware emulator |
| RR | Round Robin — fair-share scheduling policy |
| SJF | Shortest Job First — burst-based scheduling policy |
| ST | System Test — end-to-end verification across subsystems |
| TLB | Translation Lookaside Buffer — MMU cache |
| TTBR0_EL1 | Translation Table Base Register 0 for EL1 (user/low VA space) |
| UART | Universal Asynchronous Receiver-Transmitter — serial interface |
| Unity | Lightweight C unit testing framework (ThrowTheSwitch) |
| UT | Unit Test — verifies a single function or module in isolation |
| VBAR_EL1 | Vector Base Address Register — points to exception vector table |
| WB | Write-Back — normal cacheable memory attribute |
| WFE | Wait For Event — low-power idle instruction |

| Tester: Harshit | ML Runtime Developer | Test Cases: 12 | Status: ALL PASS |
| --- | --- | --- | --- |

5. Lib String

5.1 Lib String --- Summary Table

| Test Case ID | Test Description |  |  | Req ID | Type |  | Status | Date |  | Tester |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UT-STR-001 | memset fills a 32-byte buffer with non-zero pattern 0xAB |  |  | FR-018 | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-STR-002 | Second memset call can zero a previously non-zero 128-byte region |  |  | FR-018 | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-STR-003 | memcpy copies aligned 64-bit blocks without corruption |  |  | FR-018 | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-STR-004 | memcpy correctly handles unaligned source and destination offsets |  |  | FR-018 | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-STR-005 | strlen returns 0 for empty string and 6 for MiniOS literal |  |  | FR-018 | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-STR-006 | memcmp returns equality for byte-identical buffers |  |  | FR-018 | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-STR-007 | memcmp ordering semantics are preserved for first differing byte |  |  | FR-018 | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-STR-008 | memset returns destination pointer for chaining-safe semantics |  |  | FR-018 | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-STR-009 | memcpy returns destination pointer and preserves copied bytes |  |  | FR-018 | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-STR-010 | Zero-length memcpy does not alter destination buffer contents |  |  | FR-018 | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-STR-011 | Zero-length memcmp returns equality even for different buffers |  |  | FR-018 | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-STR-012 | strlen stops at first null terminator in mixed buffer |  |  | FR-018 | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| Tester: Harshit |  | ML Runtime Developer | Test Cases: 11 |  |  | Status: 6/11 PASS |  |  |

6. Command Framework

6.1 Command Framework --- Summary Table

| Test Case ID | Test Description |  |  | Req ID | Type |  | Status | Date |  | Tester |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CT-CMD-001 | CMD_Register rejects NULL name and NULL handler arguments |  |  | FR-023 | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| CT-CMD-002 | CMD_Dispatch tokenizes mixed-case command and arguments with leading/trailing whitespace |  |  | FR-023 | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| CT-CMD-003 | Unknown command path prints user-facing help hint |  |  | FR-023 | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| CT-CMD-004 | Whitespace-only input line is treated as no-op |  |  | FR-023 | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| CT-CMD-005 | Command table enforces CMD_MAX_COMMANDS upper bound |  |  | FR-023 | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| CT-CMD-006 | CMD_Register accepts NULL help pointer and stores empty help string |  |  | FR-023 | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| CT-CMD-007 | Argument cap should keep final accepted token clean when input exceeds CMD_MAX_ARGS |  |  | FR-023 | Component |  | FAIL | 09-Apr-2026 |  | Harshit |
| CT-CMD-008 | Registry should reject zero-length command names |  |  | FR-023 | Component |  | FAIL | 09-Apr-2026 |  | Harshit |
| CT-CMD-009 | Registry should reject names containing whitespace separators |  |  | FR-023 | Component |  | FAIL | 09-Apr-2026 |  | Harshit |
| CT-CMD-010 | Registry should reject duplicate command names to avoid ambiguous dispatch |  |  | FR-023 | Component |  | FAIL | 09-Apr-2026 |  | Harshit |
| CT-CMD-011 | Registry should reject case-insensitive duplicates to keep command namespace unique |  |  | FR-023 | Component |  | FAIL | 09-Apr-2026 |  | Harshit |
| Tester: Harshit |  | ML Runtime Developer | Test Cases: 6 |  |  | Status: ALL PASS |  |  |

7. ULFS Commands

7.1 ULFS Commands --- Summary Table

| Test Case ID | Test Description |  |  | Req ID | Type |  | Status | Date |  | Tester |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IT-FS-001 | FS_RegisterCommands populates command table with expected filesystem command set |  |  | FR-ULFS | Integration |  | PASS | 09-Apr-2026 |  | Harshit |
| IT-FS-002 | mkdir command success path creates directory and syncs filesystem metadata |  |  | FR-ULFS | Integration |  | PASS | 09-Apr-2026 |  | Harshit |
| IT-FS-003 | mkdir duplicate path maps filesystem error into already exists message |  |  | FR-ULFS | Integration |  | PASS | 09-Apr-2026 |  | Harshit |
| IT-FS-004 | write command followed by cat returns round-trip payload content |  |  | FR-ULFS | Integration |  | PASS | 09-Apr-2026 |  | Harshit |
| IT-FS-005 | rm on non-empty directory emits directory not empty message |  |  | FR-ULFS | Integration |  | PASS | 09-Apr-2026 |  | Harshit |
| IT-FS-006 | df command prints filesystem usage report header and utilization line |  |  | FR-ULFS | Integration |  | PASS | 09-Apr-2026 |  | Harshit |
| Tester: Harshit |  | ML Runtime Developer | Test Cases: 12 |  |  | Status: 9/12 PASS |  |  |

8. Networking

8.1 Networking --- Summary Table

| Test Case ID | Test Description |  |  | Req ID | Type |  | Status | Date |  | Tester |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CT-NET-001 | UDP bind table enforces maximum handler capacity |  |  | FR-NET-UDP | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| CT-NET-002 | UDP receive demultiplexes by destination port and forwards payload intact |  |  | FR-NET-UDP | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| IT-NET-003 | UDP send path builds correct IPv4 plus UDP wire framing |  |  | FR-NET-IPV4,FR-NET-UDP | Integration |  | PASS | 09-Apr-2026 |  | Harshit |
| CT-NET-004 | SFU serialize plus deserialize round-trip preserves header and payload fields |  |  | FR-NET-SFU | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| IT-NET-005 | Incoming SFU PING triggers outbound PONG with same request id |  |  | FR-NET-SFU | Integration |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-NET-006 | UDP receive ignores datagrams shorter than UDP header size |  |  | FR-NET-UDP | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-NET-007 | UDP send honors payload boundary at MTU-derived limit |  |  | FR-NET-UDP | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-NET-008 | SFU deserialize rejects frames whose declared payload is truncated |  |  | FR-NET-SFU | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-NET-009 | SFU checksum mismatch on receive triggers NACK with original request id |  |  | FR-NET-SFU | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-NET-010 | UDP_Bind should reject NULL callback handlers |  |  | FR-NET-UDP | Unit |  | FAIL | 09-Apr-2026 |  | Harshit |
| UT-NET-011 | UDP_Receive should reject datagrams with declared length smaller than header size |  |  | FR-NET-UDP | Unit |  | FAIL | 09-Apr-2026 |  | Harshit |
| UT-NET-012 | UDP_Receive should reject datagrams whose declared length exceeds actual frame size |  |  | FR-NET-UDP | Unit |  | FAIL | 09-Apr-2026 |  | Harshit |
| Tester: Harshit |  | ML Runtime Developer | Test Cases: 6 |  |  | Status: ALL PASS |  |  |

9. ONNX Parser

9.1 ONNX Parser --- Summary Table

| Test Case ID | Test Description |  |  | Req ID | Type |  | Status | Date |  | Tester |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CT-ONNX-PARSE-001 | Custom binary parser rejects invalid magic constant |  |  | FR-ONNX-PARSER | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| CT-ONNX-PARSE-002 | Custom binary parser rejects model declaring more than ONNX_MAX_NODES |  |  | FR-ONNX-PARSER | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| CT-ONNX-PARSE-003 | Minimal valid custom binary model with zero nodes and tensors loads successfully |  |  | FR-ONNX-PARSER | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| UT-ONNX-PARSE-004 | ONNX_LoadEmbedded rejects unsupported format enum values |  |  | FR-ONNX-PARSER | Unit |  | PASS | 09-Apr-2026 |  | Harshit |
| CT-ONNX-PARSE-009 | Custom binary parser rejects unsupported header version |  |  | FR-ONNX-PARSER | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| CT-ONNX-PARSE-010 | Custom binary parser rejects corrupt metadata bounds for tensor section |  |  | FR-ONNX-PARSER | Component |  | PASS | 09-Apr-2026 |  | Harshit |
| Tester: Harshit |  | ML Runtime Developer | Test Cases: 8 |  |  | Status: 6/8 PASS |  |  |

10. ONNX Runtime

10.1 ONNX Runtime --- Summary Table

| Test Case ID | Test Description | Req ID | Type | Status | Date | Tester |
| --- | --- | --- | --- | --- | --- | --- |
| UT-ONNX-RUNTIME-005 | ONNX_Runtime_Init validates NULL context and NULL graph pointers | FR-ONNX-RUNTIME | Unit | PASS | 09-Apr-2026 | Harshit |
| UT-ONNX-RUNTIME-006 | Runtime control flags round-trip through setter and getter APIs | FR-ONNX-RUNTIME | Unit | PASS | 09-Apr-2026 | Harshit |
| CT-ONNX-RUNTIME-007 | ExecuteNode rejects unknown operator type with explicit unsupported status | FR-ONNX-RUNTIME | Component | PASS | 09-Apr-2026 | Harshit |
| IT-ONNX-RUNTIME-008 | Inference API rejects mismatched input count at integration boundary | FR-ONNX-RUNTIME | Integration | PASS | 09-Apr-2026 | Harshit |
| UT-ONNX-RUNTIME-011 | Inference API rejects mismatched output count when output array is provided | FR-ONNX-RUNTIME | Unit | PASS | 09-Apr-2026 | Harshit |
| UT-ONNX-RUNTIME-012 | ExecuteUpTo rejects lookup of missing node names | FR-ONNX-RUNTIME | Unit | PASS | 09-Apr-2026 | Harshit |
| UT-ONNX-RUNTIME-013 | ExecuteUpTo should require exact node-name match not prefix match | FR-ONNX-RUNTIME | Unit | FAIL | 09-Apr-2026 | Harshit |
| UT-ONNX-RUNTIME-014 | ExecuteUpTo should fail when target node is not present in execution schedule | FR-ONNX-RUNTIME | Unit | FAIL | 09-Apr-2026 | Harshit |